| THE MINISTER OF INFORMATION AND COMMUNICATIONS ------- | SOCIALIST REPUBLIC OF VIET NAM Independence - Freedom – Happiness --------- |
| --- | --- |
| No. 27/2010/TT-BTTTT | Hanoi, November 24, 2010 |

CIRCULAR

PROMULGATING THE PLAN ON THE FREQUENCY BAND OF 2500-2690 MHZ FOR VIETNAM'S IMT MOBILE COMMUNICATION SYSTEM

Pursuant to the November 23, 2009 Law on Radio Frequency;
Pursuant to the Government's Decree No. 187/2007/ND-CP of December 25, 2007, defining the functions, tasks, powers and organizational structure of the. Ministry of Information and Communications;
Pursuant to the Prime Minister's Decision No. 125/2009/QD-TTg of October 23, 2009, on the plan on the national radio frequency spectra;
At the proposal of the director of the Radio Frequency Directorate,

STIPULATES:

Article 1. To promulgate the plan on the frequency band of 2500-2690 MHz for Vietnam's IMT mobile communication system with the following contents:

1. Scope of regulation:

This plan is applicable to establishing the mobile communication systems up to IMT standard of the International Telecommunications Union (ITU) in Vietnam in the frequency band of 2500-2690 MHz.

2. Contents of the plan:

a/ The frequency band of 2500-2690 MHz is reserved for IMT mobile communication system in Vietnam;

b/ The frequency band of 2500-2690 MHz is divided as follows:

Each operator of the IMT mobile communication system using Frequency Division Duplex (FDD) is entitled to one block A-A, B-B' and C-C’ as specified in the frequency band use license.

Operators of the IMT mobile communication system using Time Division Duplex (TDD) are entitled to block D as specified in the frequency band use license.

c/ The sections of 2570-2575 MHz and 2615-2620 MHz are reserved for protective band between operators using FDD and operators using TDD;

d/ Operators of the IMT mobile communication system licensed in the frequency band of blocks A-A’, B-B'; C-C and D shall coordinate with each other in using radio frequency and applying measures to prevent harmful interferences.

3. Implementation of the plan:

a/ Organizations and individuals may not use and deploy more new radio equipment in the frequency band of 2500-2690 MHz;

b/ Organizations and individuals currently allowed to use the radio communication system in the frequency band of 2500-2690 MHz not for the purposes specified in this plan may not continue getting a license extension and shall change use purposes or discontinue the use under regulations of the Ministry of Information and Communications.

Article 2. This Circular takes effect on February 1, 2011,

Article 3. The Chief of the Office, the director of the Radio Frequency Directorate, heads agencies and units under the Ministry of Information and Communications, organizations, individuals using, producing, importing or trading in radio equipment for use in Vietnam shall implement this Circular.-

|  | FOR THE MINISTER OF INFORMATION AND COMMUNICATIONS DEPUTY MINISTER Le Nam Thang |
| --- | --- |