| THE PRIME MINISTER ------- | SOCIALIST REPUBLIC OF VIET NAM Independence - Freedom – Happiness --------- |
| --- | --- |
| No. 1208/QD-TTg | Hanoi, July 21, 2011 |

DECISION

APPROVING THE NATIONAL MASTER PLAN FOR POWER DEVELOPMENT IN THE 2011-2020 PERIOD, WITH CONSIDERATIONS TO 2030

THE PRIME MINISTER

Pursuant to the December 25, 2001 Law on Organization of the Government;

Pursuant to the December 14, 2004 Electricity Law;

Considering Report No. 2068/TTr-BCT of March 11, 2011, of the Ministry of Industry and Trade and opinions of various ministries and sectors on the national master plan for power development in the 2011- 2020 period, with considerations to 2030,

DECIDES:

Article 1. To approve the national master plan for power development in the 2011-2020 period, with considerations to 2030 (Power Master Plan VII), with the following principal contents:

1. Development viewpoints:

a/ To develop the electricity sector in conformity with the national socio-economic development strategy, ensuring adequate supply of electricity for the national economy and social life;

b/ To effectively utilize domestic energy resources for electricity development in combination with the rational import of electricity and fuel, diversification of primary energy sources for electricity production, conservation of fuel and energy security for the future;

c/ To incrementally improve the quality of electricity for provision of electricity services of increasing quality. To apply electricity sale prices under the market mechanism, aiming to encourage investment in the development of the electricity sector; to encourage electricity conservation;

d/ To develop electricity along with protecting natural resources and the eco-environment, and ensuring national sustainable development;

e/ To step by step form and develop a competitive electricity market, diversifying modes of electricity investment and trading. The State shall only monopolize the electricity transmission grid in order to ensure security of the national energy system;

f/ To develop the electricity sector on the basis of rational and efficient use of primary energy resources of each region; to further step up rural electrification, ensure adequate, uninterrupted and safe supply of electricity in all regions nationwide.

2. Objectives:

a/ General objectives:

To efficiently use domestic energy resources in combination with the import of primary energy for electricity production; to adequately supply electricity of increasing quality and at reasonable prices for socio-economic development; to ensure the national energy security;

b/ Specific objectives:

- To supply adequate electricity for the domestic demand, with produced and imported electricity" reaching 194- 210 billion kWh by 2015; 330- 362 billion kWh by 2020; and 695-834 billion kWh by 2030;

- To prioritize the development of renewable energy sources for electricity production, raising the rate of electricity produced from these energy sources from 3.5% of total electricity output in 2010 to 4.5% by 2020 and 6% by 2030;

- To reduce the electricity elasticity coefficient/GDP from the current average of 2.0 to 1.5 by 2015 and 1.0 by 2020;

- To accelerate the program on electrification in rural and mountainous areas, ensuring that by 2020 most of rural households will have access to electricity.

3. The national master plan on electricity development:

a/ Master plan for development of electricity sources:

- Development orientations:

To develop electricity sources in the following orientations:

+ To develop a balanced capacity of power sources in each region: northern, central and southern, ensuring the reliability of electricity supply in each regional electricity system so as to reduce transmission losses, share reserve capacity of sources and efficiently exploit hydropower plants in all seasons;

+ To rationally develop regional power centers nationwide, aiming to ensure reliable electricity supply on site and reduce technical losses on the national electricity system as well as to ensure the economics of projects, contributing to the socio-economic development in each region and the whole country;

+ To develop new electricity sources in couple with intensive investment, technological renewal of operating power plants, meeting environmental standards; to use modern technologies for new power plants;

+ To diversify forms of investment in the development of electricity sources, aiming to boost competition and improve economic efficiency.

- The master plan for development of electricity sources:

+ To prioritize quick development of electricity sources from renewable energy (wind energy, solar energy, biomass energy, etc.), and incrementally raise the proportion of electricity produced from renewable energy sources:

To raise the total wind electricity output from the current negligible levels to around 1,000 MW by 2020 and 6,200 MW by 2030; and increase the proportion of electrical energy produced from wind power from 0.7% in 2020 to 2.4% in 2030;

+ To develop biomass electricity simultaneously with electricity generation in sugar mills, with a total output of around 500 MW by 2020, then 2,000 MW by 2030; to raise the output ratio of this electricity source from 0.6% by 2020 to 1.1% by 2030;

+ To prioritize development of hydropower sources, particularly projects of multiple purposes: flood control, water supply and electricity production; raising the hydropower output from the current 9,200 MW to 17, 400 MW by 2020.

+ To study and put pumped-storage hydropower plants into operation in conformity with the development of the electricity system, aiming to raise the operational efficiency of the system: By 2020, the total pumped-storage hydropower output will reach 1,800 MW, which will be raised to 5,700 MW by 2030;

+ To develop a rational number of thermo-power plants suitable to the supply capacity and distribution of fuel sources:

Natural gas-fired thermo-electricity: By 2020, the natural gas-fired electricity output will reach around 10,400 MW, generating 66 billion kWh, accounting for 20% of electricity production, which is oriented to rise to about 11,300 MW, generating 73.1 billion kWh of electricity, accounting for 10.5% of the electricity output.

Eastern South Vietnam: To ensure a stable gas supply for power plants in Ba Ria, Phu My and Nhon Trach.

Western South Vietnam: To expeditiously carry gas from Block B ashore in 2015 for supply to power plants in the O Mon power center with a total capacity of about 2,850 MW, bringing the total capacity of gas fired power plants in the region to 4,350 MW by 2016, which will annually consume about 6.5 billion cubic meters of gas to produce 31.5 billion kWh of electricity.

Central region: It is expected that after 2020, a 1,350-MW power plant will be developed, consuming around 1.3 billion cubic meters of gas/year.

Coal-fired thermo-electricity: TO tap to the utmost domestic coal resources for development of thermo-power plants, prioritizing the use of domestic coal for thermo-power plants in the North. By 2020, the total coal-fired thermoelectricity capacity will approximate 36,000 MW, generating some 156 billion kWh (accounting for 46.8% of electricity production) and consuming 67.3 million tons of coal. By 2030, the total coal-fired thermo-electricity capacity will be about 75,000 MW, generating some 394 billion kWh (accounting-for 56.4% of electricity production) and consuming 171 million tons of coal. Due to limited domestic coal resources, thermo-power plants operated by imported coal should be built and operated from 2015.

+ To develop nuclear power plants to ensure stable electricity supply in the future when domestic primary energy sources are exhausted:: To put Vietnam's first nuclear power generation unit into operation in 2020; by 2030, the nuclear power capacity will reach 10,700 MW, generating about 70.5 billion kWh (accounting for 10.1% of electricity production);

+ To develop power plants operated by liquefied natural gas (LNG), aiming to diversify fuel sources for electricity production, ensuring security in electricity and gas supply. By 2020, the LNG-fired electricity capacity will approximate 2,000 MW, which is expected to rise to about 6,000 MW in 2030;

+ Export and import of electricity: To efficiently exchange electrical energy with regional countries, ensuring the interests of parties involved, intensifying the exchange to maintain the safety of the system, stepping up the import of electricity from potential regions of hydropower, primarily Laos, then Cambodia and China, It is expected that the imported electricity capacity will approximate 2,200 MW by 2020,and 7,000 MW by 2030.

- Structure of electricity sources:

By 2020: The total capacity of power plants will approximate 75,000 MW, of which hydropower will account for 23.1%; pumped storage hydropower, 2.4%; coal-fired thermo-power, 48%; gas-fired thermo-power, 16.5% (including 2.6% for LNG); renewable energy-using electricity, 5.6%; nuclear power, 1.3%, and imported electricity, 3.1%.

Produced and imported electricity will approximate 330 billion kWh, of which hydropower will account for 19.6%; coal-fired thermo-power, 46.8%; gas-fired thermo-power, 24% (including 4% for LNG); renewable energy-using electricity, 4.5%; nuclear power, 2.1%, and imported electricity, 3%;

Orientations to 2030: The total capacity of power plants will approximate 146,800 MW, of which hydropower will account for 11.8%; pumped-storage hydropower, 3.9%; coal-fired thermo-power, 51.6%; gas-fired thermo-power, 11.8% (including 4.1% for LNG); renewable energy-using electricity, 9.4%; nuclear electricity, 6.6% and imported electricity, 4.9%.

Electricity output in 2030 will be 695 billion kWh, of which hydropower will represent 9.3 %; coal-fired thermo-power, 56.4%; gas-fired thermo-power, 14.4% (including 3.9% for LNG); renewable energy-using electricity sources, 6%; nuclear power, 10.1%, and imported electricity, 3.8%.

The list and schedule of operation of electricity source projects is provided in Appendices I, II and III to this Decision (not printed herein).

b/ Master plan for development of the electricity grid:

- Criteria for formulation of a master plan for development of the electricity grid:

+ The electricity transmission grid to be invested will reach reliability standard N-1 for major equipment and satisfy the quality standards set in the Regulation on electricity transmission grids;

+ The electricity grid will be developed according to technical standards applied by regional countries, ensuring connectivity and synchronism between electricity systems of Vietnam and regional countries;

+ The electricity transmission grid must have reserves, be simple and flexible, and ensure electricity quality (voltage, frequency) supplied for power load;

+ Appropriate transmission voltages will be selected on the basis of transmission capacity and distance.

- Development orientations:

+ To develop electricity transmission grids in synchronism with operation schedules of power plants to achieve investment effectiveness of the whole system;

+ To develop electricity transmission grids in conformity with the sector's development strategy, power development master plan and other local master plans;

+ To develop 220 kV and 500 kV transmission grids in order to raise electricity supply reliability and reduce electricity losses, and ensure easy mobilization of electricity sources in the rainy and dry seasons and under all modes of operation of the electricity market;

+ To develop 220 kV and 110 kV grids and perfect regional electricity networks with a view to raising reliability and stability of electricity supply, minimizing electricity losses and facilitating the transformation of medium-voltage grids into 22 kV grids and rural electrification;

+ To develop electricity transmission lines with reserves for long-term development in the future, using multi-circuit and multi-voltage poles on a single line for saving land. For cities and major load centers, electricity grid maps must have greater reserves and flexibility; to modernize and incrementally lay underground electricity grids in cities and towns, limiting adverse impacts on landscape and environment;

+ To step by step modernize electricity grids, renovate and upgrade switching, protection and automation devices of electricity grids; to study the use of FACTS and SVC devices for raising the transmission limits; to gradually modernize the control system;

+ To study the application of "Smart Grid" technology, creating interactivity between electricity-using households, electrical appliances and electricity grids for the most effective exploitation of the supply ability, aiming to reduce costs in grid development and raise the safety of electricity supply.

- Master plan for development of the electricity grid:

+ Master plan for development of ultra-high voltage transmission grids:

- The 500 kV voltage is the major ultra-high voltage transmission level in Vietnam;

To examine the possibility of developing voltages of 750 kV and 1,000 kV or DC transmission in the post-2020 period;

The 500 kV grid will be used for transmitting electricity from power centers and major power plants to major load centers in each region and for exchanging electricity among regions in order to ensure optimum operation of the electricity system.

+ Master plan for development of 220 KV transmission grids:

- Transformer stations with 2 or 3 transformers; to consider development of 4-transformer stations, GIS transformer stations and underground ones in major cities;

Newly built transmission lines must use at least double circuits; lines from major electricity sources, 500/220 kV transformer stations must be designed with at least double circuits, using phase separation conductors.

Table 1. Volume of transmission grid to be built in each period

| Item | Unit | 2011-15 | 2016-20 | 2021-25 | 2026-30 |
| --- | --- | --- | --- | --- | --- |
| 500 kV station | MVA | 17,100 | 26,750 | 24,400 | 20,400 |
| 220 kV station | MVA | 35,863 | 39,063 | 42,775 | 53,250 |
| 500 kV line | Km | 3,833 | 4,539 | 2,234 | 2,724 |
| 220kVline | Km | 10,637 | 5,305 | 5,552 | 5,020 |

+ Master plan for development of 110 kV electricity grids and distribution grids:

To develop 110 kV electricity grids and distribution grids in synchronism with electricity transmission grids in order to raise electricity supply reliability and meet the quality standards set in the Regulations on electricity distribution grids;

To apply modern technologies in order to raise the quality of electricity distribution grids, incrementally laying underground electricity grids in cities and towns to limit impacts on landscape and environment. To apply modern technological solutions in the investment and operation management to reduce electricity losses and eventually build smart grids and smart communities with a view to reducing electricity losses and increasing electricity conservation.

The list and schedule of new investment projects on electricity transmission grids are provided in Appendices IV and V to this Decision (not printed herein).

c/ Connection to electricity grids of regional countries:

- To implement the programs on electricity grid cooperation and connections with ASEAN countries and Great Mekong Sub-Region (GMS) countries;

- Connection to Laos grids:

+ Northern Lao region: By 220 kV and 500 kV voltages toward Thanh Hoa, Nho Quan (Ninh Binh province) and Son La;

+ Central and southern Lao regions: By 220 kV and 500 kV voltages toward Thach My (Quang Nam) and Pleiku (Gia Lai).

- Connection to Cambodia's grids:

+ Connection for electricity trading with Cambodia through 220 kV and 500 kV voltages, depending on capacity.

- Connection to China's grids:

+ To maintain the import of electricity by 110 kV and 220 kV voltages;

+ To study the import by 500 kV voltage or DC voltage with a total imported volume of about 2,000 - 3,000 MW.

d/ Supply of electricity to rural mountainous areas and islands:

- Objectives:

+ To make new investments with the national electricity grid or local electricity sources (small and micro hydropower plants; solar power and wind power in combination with diesel power plants) to supply electricity to rural areas. By 2015, 100% communes and 98.6% of rural households will have access to electricity. By 2020, most rural households will be supplied with electricity;

+ To renovate and upgrade rural electricity grids to set technical standards, aiming to ensure efficient supply of electricity with assured quality for needs of production development and daily life in rural areas.

- Viewpoints on development of electricity in rural and mountainous areas:

+ To step up rural electrification in order to accelerate agricultural and rural industrialization and modernization;

+ To use new and renewable energy sources for supplying electricity to deep-lying, remote, border and island areas. To adopt favorable management and investment mechanisms for the maintenance and development of electricity sources in these areas;

+ To enhance control of rural electricity prices to ensure the implementation of electricity price policies issued by the Government.

- Master plan for electricity supply in rural areas:

+ During 2011-2015:

. To invest for expansion of the national electricity grid to supply electricity for 500,000 rural households;

. To supply electricity from renewable energy sources for about 377,000 rural households.

+ During 2016-2020:

To invest in new electricity supply from the national electricity grid for 200,000 rural households;

.To supply electricity from renewable energy sources for some 231,000 rural households.

e/ Total investment capital:

The total investment capital for the whole electricity sector through 2020 is estimated at VND 929.7 trillion (equivalent to USD 48.8 billion or USD 4.88 billion per year an -average). In the 2021- 2030 period, the total investment capital is estimated at VND 1,429,3 trillion (equivalent to USD 75 billion). In the 2011-2030 period, the investment capital will approximate VND 2,359 trillion (equivalent to USD 123.8 billion), of which:

- Investment in electricity sources: The 2011-2020 period requires VND 619,3 trillion, accounting for 66.6% of the total investment capital; while the 2021 -2030 period needs VND 935.3 trillion, accounting for 65.5%$'

- Investment in electricity grids: The 2011-2020 period requires VND 210.4 trillion, accounting for 33.4% of the total investment capital; and the 2021- 2030 period needs VND 494 trillion, accounting for 34.5%.

4. Solutions for implementation of the master plan

a/ Solutions for assuring electricity supply security:

- The Electricity Vietnam (EVN), Vietnam Oil and Gas Group (Petro Vietnam) and Vietnam National Coal- Mineral Industries Holding Corporation (Vinacomin) shall assume the prime responsibility for developing electricity sources; the National Electricity Transmission Corporation shall assume the prime responsibility for assuring the development of the national transmission system;

- To actively seek additional sources of gas to existing ones which will decrease and be exhausted in the coming period. To speed up negotiations with other countries for conclusion of stable and long-term contracts on import of coal for thermo-power plants;

- To speed up the development of the nuclear energy industry and the building of nuclear power plants. To coordinate with other countries and international organizations in developing the use of nuclear energy, incrementally master the technology and develop nuclear power for peaceful purposes;

- To adopt policies on financial incentives and expand international cooperation to intensify exploration work for increasing coal, gas and renewable energy deposits and exploitation capability, ensuring security in the supply of fuel for electricity production.

b/ Solutions for creating investment capital for development of the electricity sector:

- To incrementally raise the capability for mobilization of finance from enterprises in the electricity sector through such solutions as improving, the operational efficiency and efficacy of enterprises in the electricity sector, ensuring capital accumulation and ratio of own capital for development investment as required by domestic and international financial institutions. Eventually, the main source of funding electricity works will be capital accumulated by enterprises;

- To develop economic groups and corporations operating in the electricity sector with high financial ratings in order to reduce costs of raising capital for electricity projects, which will be able to raise capital without the Government's guarantee;

- To raise further capital through issuing bonds at home and abroad for investment in electricity works, applying the measure of converting domestic savings into investment in infrastructure. At the initial stage, the State shall guarantee the issue of bonds for key and urgent electricity projects;

- To enter into domestic and foreign joint ventures to attract domestic and foreign investors into the development of electricity projects;

- To equitize enterprises in the electricity sector in which the State does not need to hold 100% capital;

- To further attract foreign direct investment (FDI) in developing electricity projects. To prioritize FDI projects for which payments can be made in VND or in barter without the Government's guarantee;

- To further attract funds from overseas, including preferential official development assistance capital, non-preferential official development assistance capital, commercial overseas loans, etc.

c/ Solutions regarding electricity prices:

- To apply electricity sale prices under the State-regulated market mechanism, ensuring harmonious combination between the political- economic-social objectives of the State and the production-business-financial autonomy objectives of enterprises in the electricity sector; Electricity sale prices should stimulate electricity development, create a favorable environment for attracting investment and encouraging competition at the stages of production, transmission, distribution and use of electricity;

- Electricity sale prices must ensure recovery of costs and reasonable profits (investment for expanded production) to ensure that financial autonomy of enterprises in the electricity sector;

- To improve and perfect the current electricity price tariff toward:

+ Making adjustments to electricity sale prices according to changes in fuel prices, exchange rates and structure of electricity production;

+ Gradually reducing and eventually eliminating cross offsetting among customer groups and among regions; studying the application of different seasonal and regional electricity sale price tariffs;

+ Supplementing a two-component electricity price tariff: capacity price and electricity price, first of all applying it to major electricity users.

- Electricity sale prices should take into account the particularities of regions and regional inhabitants: border, island, rural area, mountainous area, etc., with necessary price subsidies and tax subsidies in order to reduce the gap in electricity consumption, socioeconomic development and urbanization among regions and segments of the population, between mountainous and delta regions and between rural and urban areas;

- Electricity prices will be incrementally adjusted to achieve the long-term marginal cost of the electricity system by 2020, equivalent to 8- 9 US cents/kWh, ensuring sustainable development of the electricity sector and meeting the demand for electricity system development;

- The pricing of electricity must aim to preserve energy and avoid waste of non-renewable energy sources, encourage the rational use of various types of energy and the use of domestic energy, reducing reliance on imported energy.

d/ Solutions for renovating management and raising effectiveness of electricity activities:

- To study and apply appropriate management models in the electricity sector to raise productivity, speed up investment in electricity projects; to raise reliability in the operation of the electricity system;

- To restructure the electricity sector in order to step by step form an healthy competitive electricity market on the basis of electricity supply security, aiming to reduce costs and raise efficiency in electricity production and business activities, make price signals public and transparent to attract investment and develop a sustainable electricity sector.

e/ Environmental protection solutions:

- To implement the law on assessment of environmental impacts of projects and assessment of strategic environment of master plans;

- To consolidate environmental management organizations of state management agencies in charge of environment and enterprises in the electricity sector;

- To fully perform environmental monitoring and observation, measurement and management of environmental criteria; to inspect and examine the implementation of regulations on environmental protection by electricity enterprises;

- To effectively implement the electricity conservation program and raise efficacy in the areas of production, transmission, distribution and use of electricity;

- To combine electricity sector development with environmental protection:

+ The State shall adopt policies on investment and tax supports for development of forms of energy with minimal environmental impacts, contributing to environmental improvement: new and renewable energies; use of agricultural waste and forestry and urban wastes to generate electricity,...;

+ To strictly manage electricity generation technologies in environmental aspects. Selected technologies must be advanced and highly performance and low environmental impacts;

- To adopt mechanisms to attract investment capital for environmental protection from all economic sectors, particularly financial assistance from overseas for environmental protection;

- To formulate financial regulations on the electricity, sector's environment, fully and correctly accounting environmental expenses in investment and production costs;

- To encourage cooperation between large electricity production and consumption enterprises and foreign countries in the implementation of clean development mechanism (CDM) in the form of developing new and renewable energies; raising the efficiency of energy use and energy conservation projects.

f/ Solutions and policies for scientific and technological development:

- To complete, modernize and innovate electricity equipment and technologies to develop energy for the immediate and long-term future;

- To determine appropriate models and roadmaps for electricity source technologies and grids, ensuring stable and suitable development under Vietnam's conditions of natural resource potential and investment capability, affordable price and environmental protection;

- New energy facilities must be built with modern technologies suitable to Vietnam's economic conditions; to incrementally upgrade and renovate existing facilities to ensure technical, economic and environmental standards;

- To combine new modem technologies with improving existing ones to raise efficiency and energy saving;

- To encourage the use of new technologies at thermo-power plants: Spraying combustion chamber, boiling bed, above-limit steam parameters, cycle of mixed-gas turbine; waste treatment, etc. in order to raise environmental protection efficiency;

- To renovate and upgrade electricity transmission and distribution grids in order to reduce losses and ensure safety and reliability;

- To modernize moderation, operation, communication, control and automation systems to serve the moderation of domestic electricity grids and regional connection;

- To step by step apply recommended and compulsory measures to renew technologies and equipment of power-intensive industries (steel, cement and chemicals); to limit and eventually ban the import of used and low-efficient equipment in electricity production and use.

g/ Human resource development solutions:

- Human resource training: To develop specialized electricity schools, striving to build a number of them up to international standards; to develop uniform standard curricula on specialized disciplines;

- To concentrate on training human resources for key activities in the areas of electricity production, transmission and distribution. To attach importance to vocational training for skilled technicians and operators fully capable of mastering and expertly handling modern technical equipment and technologies. To organize retraining for technical and managerial staff, gradually raising the training quality to regional and world levels;

- To renovate the curriculum on training of human resources in the electricity sector, diversify forms of training and training linked with actual production; to attach importance to the recruitment of scientific and managerial staff for overseas training in spearhead fields. To organize additional training and in-advance training in missing or weak sectors, particularly nuclear power and new energies. To work out satisfactory treatment mechanisms to attract quality human resources;

- To organize, restructure and reorganize production models in a scientific and rational manner to ensure effective labor use and high productivity.

h/ To build and develop the electrical engineering industry and localization:

- To increase investment in, and diversify sources of capital, attract foreign countries' participation in the research, design and manufacture of equipment and parts for the electricity sector. Electrical equipment- and accessory productions establishments strive to turn out products of international standards.

- To form a number of electrical equipment research, designing and manufacturing complexes with the core being domestic mechanical manufacturing enterprises;

- To build modern centers for electrical equipment repair and maintenance to be able to repair and verify electrical equipment;

- To renew and modernize existing electrical engineering plants, to expand joint ventures and build new factories, creating electrical equipment manufacturing zones, striving for the domestic manufacturing of most electricity grid, transmission and distribution equipment instead of import and 50-60% of equipment for coal-operated thermo-power plants by 2020; and 60-70% of equipment for coal-operated thermo-power plants and 40- 50% of equipment for nuclear power plants by 2030.

i/ Solutions for electricity conservation:

- To enhance the communication, dissemination and implementation of the Law on Energy Conservation, aiming to raise energy use efficiency in general and electricity consumption in particular in production, business and household consumption;

- To widely implement, and increase the effectiveness of, the national target program on electricity conservation for saving 5- 8% and by 2020 and 8-10% of total electricity consumption.

Article 2. Tasks of concerned ministries, sectors, localities and units

1. The Ministry of Industry and Trade shall:

a/ Periodically inspect and urge investors and contractors assigned to implement projects on the master plan's lists and concerned units to promptly direct and solve problems in the course of Investment and construction with a view to strictly keeping to the approved schedules of projects and achieving their investment effectiveness. To report on delayed projects to the Prime Minister for consideration and handling;

b/ Strictly monitor the electricity supply-demand, .progress of implementation of electricity source and grid projects in order to decide and; adjust the schedules of projects in the approved master plan or consider and report to the Prime Minister for permission to add new projects to the master plan or remove unnecessary projects from the master plan in accordance with the practical requirements of socio-economic development in each period;

c/ Direct the formulation, appraisal and approval of plans on locations, detailed plans of thermo-power centers, and plans on hydropower on rivers. Direct the development or import of gas and new coal resources for electricity production, industry and other essential demands;

d/ Publicize lists of projects in the approved plans and select investors for projects for submission to the Prime Minister for approval; to assume the prime responsibility for formulation and submission to the Prime Minister for approval of mechanisms to select investors for developing new electricity sources and mechanisms for implementation management to ensure the project investment, construction and operation in accordance with the approved schedule;

e/ Organize open international biddings to select investors for electricity source projects to be implemented in BOT form. In case of necessity to appoint contractors, the Ministry of Industry and Trade shall assume the prime responsibility for, and coordinate with concerned ministries in, considering and submitting them to the Prime Minister for decision;

f/ Formulate plans for raising capital for electricity development and report them to the Prime Minister in October 2011;

g/ Study and submit to the Prime Minister a plan on development of smart grids in December 2011;

h/ Assume the prime responsibility for, and coordinate with concerned ministries, sectors and localities in, formulating mechanisms and policies to encourage development investment in projects on new and renewable energies, and the national target program on development of renewable energy;

i/ Assume the prime responsibility for, and coordinate with ministries and sectors in, negotiating on and concluding electricity cooperation and exchange agreements with neighboring countries and Vietnam's participation in the connected electricity systems among the countries in the Mekong sub-region;

j/ Direct and urge localities and investors to properly implement rural electricity development projects according to approved plans and schedules;

k/ Complete the necessary conditions (legal, technical infrastructure) for the formation and operation of a competitive electricity market;

1/ Assume the prime responsibility for formulation and submission to the Prime Minister for approval of programs on research, manufacture and localization of, equipment for coal- operated thermo-power plants, hydropower and nuclear power plants;

m/ Develop mechanisms for development of the energy market, balance the use of primary energy sources for electricity production, prioritize domestic coal and gas resources for electricity projects;

n/ Assume the prime responsibility for, and coordinate with ministries, sectors and localities in, implementing the 2011-2015 program on electricity saving;

o/ Draft a decision on establishment of the State Steering Committee for the national master plan for power development and submit it to the Prime Minister for approval.

2. The Ministry of Planning and Investment shall:

a/ Formulate mechanisms and policies to attract foreign investment, ODA capital and private investment capital for synchronous, balanced and sustainable development of the electricity sector;

b/ Assume the prime responsibility for, and coordinate with the Ministry of Industry and Trade in, registering, arranging and additionally allocating adequate budget funds for the formulation and publicity of the master plan.

3. The Ministry of Finance shall assume the prime responsibility for, and coordinate with concerned ministries and sectors in, formulating financial mechanisms and capital mechanisms for investment in the development of the electricity sector under the approved master plan; and coordinate with the Ministry of Industry and Trade in formulating policies on electricity prices under the market mechanism.

4. The State Bank of Vietnam shall assume the prime responsibility for, and coordinate with concerned ministries and sectors in. formulating appropriate mechanisms and policies to enhance the capacity of banks to ensure adequate supply of funds for electricity projects under the approved master plan.

5. Vietnam Electricity (EVN) shall:

a/ Invest in, and put into operation the assigned electricity source projects and electricity transmission projects according to the approved schedules. Take responsibility for the purchase of electricity from electricity sources, import of electricity, and management and operation of electricity transmission grids and distribution grids in order to property perform its key role in ensuring the national electricity supply;

b/ Draw up plans on locations and detailed plans for coal-fired thermo-electricity centers as assigned to it in the master plan, and submit them to the Ministry of Industry and Trade for approval;

c/ Assume the prime responsibility for .investment and construction of infrastructure facilities of thermo-electricity centers of which the Electricity Vietnam will partially invest in electricity source projects;

d/ Apply solutions for further reducing electricity losses, implement the programs on electricity conservation in production, transmission and distribution for sustainable development;

e/ Assign the National Electricity Transmission Corporation to invest in 500 kV and 22frfcV electricity transmission grid projects in Electricity Master Plan VII.

6. The Vietnam Oil and Gas Group (Petro VietNam) shall:

a/ Invest in and operate the assigned electricity source projects according to the approved schedules;

b/ Develop and operate new oil and gas fields in synchronism with the schedule of putting into operation the power plants and ensure adequate supply of gas for these plants approved in the master plan. Assume the prime responsibility for formulating a plan on import of liquefied natural gas for electricity generation in conformity to the schedule of development of power plants in the master plan, and submit it to the Mia is try of Industry and Trade for approval

7. The Vietnam National Coal- Mineral Industries Holding Corporation (Vinacomin) shall:

a/ Invest in and operate the assigned electricity source projects according to the approved schedule;

b/ Invest in the exploitation of new coal mines, acting as a coordinator for in the import of coal to properly perform its key role in ensuring the supply of coal for electricity production and other demands of the national economy.

8. Provincial-level People's Committees shall:

a/ Arrange land areas in their local land use planning for electricity source and electricity transmission grid works approved in this master plan and electricity distribution grid works approved in local electricity development plans;

b/ Assume the prime responsibility for, and closely coordinate with investors in, ground clearance, compensation, population relocation and resettlement for electricity source and grid projects according to the approved schedule.

Article 3. This Decision takes effect on the date of its signing.

Ministers, heads of ministerial-level agencies, chairmen of provincial-level People's Committees, the Members' Councils, chairmen of the Members' Councils, directors-general of Vietnam Electricity, Vietnam Oil and Gas Group, Vietnam National Coal- Mineral Industries Holding Corporation and concerned agencies shall implement this Decision.-

|  | PRIME MINISTER Nguyen Tan Dung |
| --- | --- |

APPENDIX I:

LIST OF POWER GENERATION PROJECTS EXPECTED TO BE COMMENCED OVER THE PERIOD FROM 2011 TO 2020 
(Enclosed with the Decision No. 1208/QĐ-TTg dated July 21, 2011 of the Prime Minister)

| No | Project name | Installed capacity (MW) | Investor |
| --- | --- | --- | --- |
|  | Projects expected to be commenced in 2011 | 4187 |  |
| 1 | Son La Hydroelectric Power Station (HPS) #2, 3, 4 | 1200 | EVN |
| 2 | Nam Chien HPS #1 | 100 | Song Da Corporation |
| 3 | Na Le HPS #1, 2 | 90 | LICOGI |
| 4 | Ngoi Phat HPS | 72 | IPP |
| 5 | A Luoi HPS #1, 2 | 170 | Central Hydropower joint-stock company |
| 6 | Song Tranh 2 HPS #2 | 95 | EVN |
| 7 | An Khe – Kanak HPS | 173 | EVN |
| 8 | Se San 4A HPS | 63 | Se San 4A Hydropower joint-stock company |
| 9 | Dak My 4 HPS | 190 | IDICO |
| 10 | Se Kaman 3 HPS (in Laos) | 250 | Viet – Lao joint-stock company |
| 11 | Dak Rtih HPS | 144 | Construction Corporation no 1 |
| 12 | Dong Nai 3 HPS #2 | 90 | EVN |
| 13 | Dong Nai 4 HPS #1 | 170 | EVN |
| 14 | Uong Bi MR Thermoelectric Power Plant (TPP) #2 | 300 | EVN |
| 15 | Cam Pha II TPP | 300 | TKV |
| 16 | Nhon Trach 2 Combined Cycle Plant (CCP) | 750 | PVN |
|  | Wind power and other renewable energy | 30 |  |
|  | Projects expected to be commenced in 2012 | 2805 |  |
| 1 | Son La HPS #5, 6 | 800 | EVN |
| 2 | Dong Nai 4 HPS #2 | 170 | EVN |
| 3 | Nam Chien HPS #1 | 100 | Song Da Corporation |
| 4 | Ban Chat HPS #1, 2 | 220 | EVN |
| 5 | Hua Na HPS #1, 2 | 180 | Hua Na Hydropower joint-stock company |
| 6 | Nho Que HPS 3 #1, 2 | 110 | Bitexco joint-stock company |
| 7 | Khe Bo HPS #1, 2 | 100 | Hydropower joint-stock company |
| 8 | Ba Thuoc II HPS #1, 2 | 80 | IPP |
| 9 | Dong Nai 2 HPS | 70 | IPP |
| 10 | Dam Bri HPS | 75 | IPP |
| 11 | An Khanh I TPP #1 | 50 | An Khanh electricity joint-stock company |
| 12 | Vung Ang I TPP #1 | 600 | PVN |
| 13 | Formosa TPP #2 | 150 | Hung Nghiep Formosa Co., Ltd. |
|  | Wind power and other renewable energy | 100 |  |
|  | Projects expected to be commenced in 2013 | 2105 |  |
| 1 | Nam Na 2 HPS | 66 | IPP |
| 2 | Dak Rinh HPS #1, 2 | 125 | PVN |
| 3 | Sre Pok 4A HPS | 64 | Buon DonHydropower joint-stock company |
| 4 | Hai Phong II TPP #1 | 300 | EVN |
| 5 | Mao Khe TPP #1, 2 | 440 | TKV |
| 6 | An Khanh I TPP #2 | 50 | An Khanh electricity joint-stock company |
| 7 | Vung Ang I TPP #2 | 600 | PVN |
| 8 | Nghi Son I TPP #1 | 300 | EVN |
| 9 | Nong Son TPP | 30 | TKV |
|  | Wind power and other renewable energy | 130 |  |
|  | Projects expected to be commenced in 2014 | 4279 |  |
| 1 | Nam Na 3 HPS | 84 | IPP |
| 2 | Yen Son HPS | 70 | Binh Minh Construction & Tourism joint-stock company |
| 3 | Thuong Kontum HPS #1, 2 | 220 | Vinh Son Song Hinh hydropower joint-stock company |
| 4 | Dak Re HPS | 60 | Thien Tan Hydropower joint-stock company |
| 5 | Nam Mo HPS (in Laos) | 95 | IPP |
| 6 | Hai Phong 2 TPP #2 | 300 | EVN |
| 7 | Nghi Son I TPP #2 | 300 | EVN |
| 8 | Thai Binh II TPP #1 | 600 | PVN |
| 9 | Quang Ninh II TPP #1 | 300 | EVN |
| 10 | Vinh Tan II TPP #1, 2 | 1200 | EVN |
| 11 | O Mon I TPP #2 | 330 | EVN |
| 12 | Duyen Hai I TPP #1 | 600 | EVN |
|  | Wind power and other renewable energy | 120 |  |
|  | Projects expected to be commenced in 2015 | 6540 |  |
| 1 | Huoi Quang HPS #1, 2 | 520 | EVN |
| 2 | Dong Nai 5 HPS | 145 | TKV |
| 3 | Dong Nai 6 HPS | 135 | Duc Long Gia Lai Co. |
| 4 | Se Ka man 1 HPS (in Laos) | 290 | Viet – Lao joint-stock company |
| 5 | Quang Ninh II TPP #2 | 300 | EVN |
| 6 | Thai Binh II TPP #2 | 600 | PVN |
| 7 | Mong Duong II TPP #1,2 | 1200 | AES/BOT |
| 8 | Luc Nam TPP #1 | 50 | IPP |
| 9 | Duyen Hai III TPP #1 | 600 | EVN |
| 10 | Long Phu I TPP #1 | 600 | PVN |
| 11 | Duyen Hai I TPP #2 | 600 | EVN |
| 12 | O Mon III CCP | 750 | EVN |
| 13 | Cong Thanh TPP #1, 2 | 600 | Cong Thanh electricity joint-stock company |
|  | Wind power and other renewable energy | 150 |  |
|  | Projects expected to be commenced in 2016 | 7136 |  |
| 1 | Lai Chau HPS #1 | 400 | EVN |
| 2 | Trung Son HPS #1, 2 | 260 | EVN |
| 3 | Song Bung 4 HPS | 156 | EVN |
| 4 | Song Bung 2 HPS | 100 | EVN |
| 5 | Dak My 2 HPS | 98 | IPP |
| 6 | Dong Nai 6A HPS | 106 | Duc Long Gia Lai Co. |
| 7 | Hoi Xuan HPS | 102 | IPP |
| 8 | Se Kaman 4 HPS (in Laos) | 64 | BOT |
| 9 | Ha Se San 2 HPS (50% invested by Cambodia) | 200 | EVN - BOT |
| 10 | Mong Duong I TPP #1 | 500 | EVN |
| 11 | Thai Binh I TPP #1 | 300 | EVN |
| 12 | Hai Phong TPP #1 | 600 | Jak Resourse – Malaysia/BOT |
| 13 | An Khanh II TPP #1 | 150 | An Khanh electricity joint-stock company |
| 14 | Long Phu I TPP #2 | 600 | PVN |
| 15 | Vinh Tan I TPP #1, 2 | 1200 | CSG/BOT |
| 16 | Duyen Hai III TPP #2 | 600 | EVN |
| 17 | O Mon IV CCP | 750 | EVN |
| 18 | O Mon II CCP | 750 | BOT |
|  | Wind power and other renewable energy | 200 |  |
|  | Projects expected to be commenced in 2017 | 6775 |  |
| 1 | Lai Chau HPS #2, 3 | 800 | EVN |
| 2 | Se San 3A, 3B HPS | 105+100 | Song Da Corporation |
| 3 | Thang Long TPP #1 | 300 | Thang Long electricity joint-stock company |
| 4 | Mong Duong I TPP #2 | 500 | EVN |
| 5 | Thai Binh I TPP #2 | 300 | EVN |
| 6 | Hai Phong TPP #2 | 600 | Jak Resourse – Malaysia/BOT |
| 7 | Nghi Son II TPP #1, 2 | 1200 | BOT |
| 8 | An Khanh II TPP #2 | 150 | An Khanh electricity joint-stock company |
| 9 | Van Phong I TPP #1 | 660 | Sumimoto - Hanoinco/BOT |
| 10 | Vinh Tan VI TPP #1 | 600 | EVN |
| 11 | Vinh Tan III TPP #1 | 660 | Vinh Tan 3 Energy JSC/ BOT |
| 12 | Song Hau I TPP #2 | 600 | PVN |
|  | Wind power and other renewable energy | 200 |  |
|  | Projects expected to be commenced in 2018 | 7842 |  |
| 1 | Bao Lam HPS | 120 | Song Da Corporation |
| 2 | Nam Sum 1 HPS (in Laos) | 90 | Sai Gon Invest |
| 3 | Se Kong HPS (in Laos) | 192 | EVN - BOT |
| 4 | Na Duong II TPP #1,2 | 100 | TKV |
| 5 | Luc Nam TPP #2 | 50 | IPP |
| 6 | Vung Ang II TPP #1 | 600 | VAPCO/BOT |
| 7 | Quang Trach I TPP #1 | 600 | PVN |
| 8 | Nam Dinh I TPP #1 | 600 | Tai Kwang – Korea/BOT |
| 9 | Van Phong I TPP #2 | 660 | Sumimoto - Hanoinco/BOT |
| 10 | Song Hau I TPP #2 | 600 | PVN |
| 11 | Son My II CCP #1, 2, 3 | 1170 | (IP – Sojizt - Pacific)/BOT |
| 12 | Duyen Hai II TPP #1 | 600 | Janakuasa/BOT |
| 13 | Vinh Tan III TPP #2 | 660 | Vinh Tan 3 Energy JSC/ BOT |
| 14 | Vinh Tan VI TPP #2 | 600 | EVN |
| 15 | Power imported from China | 1000 | By negotiation |
|  | Wind power and other renewable energy | 200 | IPP |
|  | Projects expected to be commenced in 2019 | 7015 |  |
| 1 | Bac Ai Hydro accumulation power plan (HAPP) #1 | 300 | EVN |
| 2 | Dong Phu Yen HAPP #1 | 300 | Xuan Thien Co. |
| 3 | Nam Sum 3 HPS (in Laos) | 196 | Sai Gon Invest |
| 4 | Vinh Son II HPS | 80 | IPP |
| 5 | Vung Ang II TPP #2 | 600 | VAPCO/BOT |
| 6 | Quang Trach I TPP #2 | 600 | PVN |
| 7 | Nam Dinh I TPP #2 | 600 | Tai Kwang – Korea/BOT |
| 8 | Thang Long TPP #2 | 300 | Thang Long electricity joint-stock company |
| 9 | Quang Tri TPP #1 | 600 | IPP/BOT |
| 10 | Duyen Hai II TPP #2 | 600 | Janakuasa/BOT |
| 11 | Duyen Hai III TPP #3 (MR) | 600 | EVN |
| 12 | Kien Luong I TPP #1 | 600 | Tan Tao |
| 13 | Son My I CCP #4, 5 | 780 | (IP – Sojizt - Pacific)/BOT |
|  | Hiep Phuoc TPP shall be stopped. | -375 |  |
| 14 | Power imported from China | 1000 | By negotiation |
|  | Wind power and other renewable energy | 230 | IPP |
|  | Projects expected to be commenced in 2020 | 5610 |  |
| 1 | Dong Phu Yen HAPP #2, 3 | 600 | Xuan Thien Co. |
| 2 | Bac Ai HAPP #2, 3 | 600 | EVN |
| 3 | Nam Mo I HPS (in Nam Kan - Laos) | 72 | EVNI |
| 4 | Quang Tri TPP #2 | 600 | IPP/BOT |
| 5 | Central CCP #1 (in Quang Tri or Quang Ngai) | 450 |  |
| 6 | Ninh Thuan I Nuclear Power Plant (NPP) #1 | 1000 | EVN |
| 7 | Ninh Thuan I NPP II #1 | 1000 | EVN |
| 8 | Vinh Tan III TPP #3 | 660 | Vinh Tan 3 Energy JSC/ BOT |
| 9 | Kien Luong I TPP #2 | 600 | Tan Tao |
|  | Thu Duc TPP shall be stopped. | -272 |  |
|  | Wind power and other renewable energy | 300 |  |

APPENDIX II:

LIST OF POWER GENERATION PROJECTS EXPECTED TO  BE COMMENCED OVER THE PERIOD FROM 2021 TO 2030 
(Enclosed with the Decision No. 1208/QĐ-TTg dated July 21, 2011 of the Prime Minister)

| No | Project name | Installed capacity (MW) | Investor |
| --- | --- | --- | --- |
|  | Projects expected to be commenced in 2021 | 5925 |  |
| 1 | Dong Phu Yen HAPP #4 | 300 | Xuan Thien Co. |
| 2 | Bac Ai HAPP #4 | 300 | EVN |
| 3 | Ha Se San 1 HPS (in Cambodia) | 90 | EVNI |
| 4 | Se Kong HPS (in Cambodia) | 150 | EVNI |
| 5 | Hai Phong III TPP #1 | 600 | TKV |
| 6 | Van Phong II TPP #1 | 660 |  |
| 7 | Son My II CCP #1, 2 | 780 |  |
| 8 | Ninh Thuan I NPP #2 | 1000 |  |
| 9 | Ninh Thuan II NPP #2 | 1000 |  |
| 10 | Power imported from China | 1000 |  |
|  | Ninh Binh I TPP shall be stopped. | -100 |  |
|  | Uong Bi I TPP shall be stopped. | -105 |  |
|  | Can Tho TPP shall be stopped. | -150 |  |
|  | Wind power and other renewable energy | 400 |  |
|  | Projects expected to be commenced in 2022 | 5750 |  |
| 1 | Nam Theun I HPS (in Laos) | 400 | EVN |
| 2 | Hai Phong III TPP #2 | 600 | TKV |
| 3 | Cam Pha III TPP #1, 2 | 270 | TKV |
| 4 | Quynh Lap I TPP #1 | 600 | TKV |
| 5 | Long Phu II TPP #1 | 600 | Song Da Corporation |
| 6 | Van Phong II TPP #2 | 660 |  |
| 7 | Son My II CCP #3, 4, 5 | 1170 |  |
| 8 | NPP no III #1 | 1000 | EVN |
|  | Wind power and other renewable energy | 450 |  |
|  | Projects expected to be commenced in 2023 | 4530 |  |
| 1 | Ha Se San 3 HPS (in Cambodia) | 180 | BOT |
| 2 | Quang Trach II TPP #1 | 600 |  |
| 3 | Quynh Lap I TPP #2 | 600 | TKV |
| 4 | Central CCP #2 (in Quang Tri or Quang Ngai) | 450 |  |
| 5 | Kien Luong II TPP #1 | 600 |  |
| 6 | Long Phu II TPP #2 | 600 | Song Da Corporation |
| 7 | NPP no III #2 | 1000 | EVN |
|  | Wind power and other renewable energy | 500 |  |
|  | Projects expected to be commenced in 2024 | 4600 |  |
| 1 | Northern HAPP #1 | 300 |  |
| 2 | Don Duong HAPP #1, 2 | 600 | EVN |
| 3 | Quang Trach II TPP #2 | 600 |  |
| 4 | Phu Tho TPP #1 | 300 |  |
| 5 | Central CCP #3 (in Quang Tri or Quang Ngai) | 450 |  |
| 6 | Long An TPP #1, 2 | 1200 |  |
| 7 | Kien Luong II TPP #2 | 600 |  |
|  | Wind power and other renewable energy | 550 |  |
|  | Projects expected to be commenced in 2025 | 6100 |  |
| 1 | Northern HAPP II #2* | 300 |  |
| 2 | Don Duong HAPP #3, 4 | 600 | EVN |
| 3 | Hai Phong III TPP #3, 4 | 1200 | TKV |
| 4 | Nam Dinh II TPP #1 | 600 | BOT |
| 5 | Phu Tho TPP #2 | 300 |  |
| 6 | Long Phu III TPP #1 | 1000 | PVN |
| 7 | Southern CCP #1, 2 | 1500 |  |
|  | Wind power and other renewable energy | 600 |  |
|  | Projects expected to be commenced in 2026 | 5550 |  |
| 1 | Northern HAPP II #3 | 300 |  |
| 2 | Vung Ang III TPP #1 | 600 | BOT |
| 3 | Nam Dinh II TPP #2 | 600 | BOT |
| 4 | Bac Gian TPP #1 | 300 |  |
| 5 | Binh Dinh I Coal power plant (CPP) #1 | 600 |  |
| 6 | Long Phu III TPP #2 | 1000 | PVN |
| 7 | NPP no IV #1 | 1000 |  |
| 8 | Power imported from Laos | 550 |  |
|  | Wind power and other renewable energy | 600 |  |
|  | Projects expected to be commenced in 2027 | 6350 |  |
| 1 | Vung Ang III TPP #2, 3 | 1200 | BOT |
| 2 | Bac Giang TPP #2 | 300 |  |
| 3 | Kien Luong III TPP #1 | 1000 |  |
| 4 | Song Hau II TPP #1 | 1000 |  |
| 5 | Binh Dinh I CPP #2 | 600 |  |
| 6 | NPP no IV #2 | 1000 |  |
| 7 | Power imported from Laos | 550 |  |
|  | Wind power and other renewable energy | 700 |  |
|  | Projects expected to be commenced in 2028 | 7450 |  |
| 1 | Ninh Son HAPP #1 | 300 |  |
| 2 | Vung Ang III TPP #4 | 600 | BOT |
| 3 | Quynh Lap II TPP #1, 2 | 1200 |  |
| 4 | Song Hau II TPP #2 | 1000 |  |
| 5 | Kien Luong III TPP #2 | 1000 |  |
| 6 | Bac Lieu CPP #1, 2 | 1200 |  |
| 7 | Central NPP I #1 | 1350 |  |
|  | Wind power and other renewable energy | 800 |  |
|  | Projects expected to be commenced in 2029 | 9950 |  |
| 1 | Ninh Son HAPP #2, 3 | 600 |  |
| 2 | Yen Hung TPP #1, 2 | 1200 |  |
| 3 | Uong Bi III TPP #1, 2 | 1200 |  |
| 4 | Song Hau III TPP #1, 2 | 2000 |  |
| 5 | Binh Dinh II CPP #1, 2 | 2000 |  |
| 6 | An Giang CPP #1, 2 | 2000 |  |
|  | Wind power and other renewable energy | 950 |  |
|  | Projects expected to be commenced in 2030 | 9800 |  |
| 1 | Ninh Son HAPP #4 | 300 |  |
| 2 | Northern CPP 1000MW #1, 2 | 2000 |  |
| 3 | Southern CPP 1000MW #1, 2, 3, 4, 5 | 5000 |  |
| 4 | Central NPP I #2 | 1350 |  |
|  | Wind power and other renewable energy | 1150 |  |

APPENDIX III

LIST OF MODERATE AND MINOR HYDROELECTRIC POWER GENERATION PROJECTS
(Enclosed with the Decision No. 1208/QĐ-TTg dated July 21, 2011 of the Prime Minister)

| No | Project name | Installed capacity (MW) | Investor |
| --- | --- | --- | --- |
|  | Projects expected to be commenced in 2011 | 280,5 |  |
| 1 | Muong Hum HPS | 32 | Son Vu Co. |
| 2 | Su Pan 2 HPS | 34,5 | IPP |
| 3 | Huong Dien HPS #3 | 27 | HD investment joint-stock company |
| 4 | Song Giang 2 HPS | 37 | IPP |
| 5 | Minor HPSs | 150 |  |
|  | Projects expected to be commenced in 2012 | 657 |  |
| 1 | Ta Thang HPS | 60 | IPP |
| 2 | Nam Phang HPS | 36 | IPP |
| 3 | Nam Toong HPS | 34 | IPP |
| 4 | Ngoi Hut 2 HPS | 48 | IPP |
| 5 | Nam Muc HPS | 44 | IPP |
| 6 | Van Chan HPS | 57 | IPP |
| 7 | Song Bung 4A HPS | 49 | IPP |
| 8 | Song Tranh 3 HPS | 62 | IPP |
| 9 | Nho Que 1 HPS | 32 | IPP |
| 10 | Chiem Hoa HPS | 48 | IPP |
| 11 | Song Bung 5 HPS | 49 | IPP |
| 12 | Minor HPSs | 138 |  |
|  | Projects expected to be commenced in 2013 | 401,5 |  |
| 1 | Ba Thuoc I HPS | 60 | IPP |
| 2 | Nam Pan 5 HPS | 34,5 | IPP |
| 3 | Nam Cun HPS | 40 | IPP |
| 4 | Song Bac HPS | 42 | IPP |
| 5 | Nhan Hac HPS | 45 | IPP |
| 6 | Minor HPSs | 180 |  |
|  | Projects expected to be commenced in 2014 | 655 |  |
| 1 | Nho Que 2 HPS | 48 | IPP |
| 2 | Long Tao HPS | 42 | IPP |
| 3 | Bac Me HPS | 45 | IPP |
| 4 | Chi Khe HPS | 41 | IPP |
| 5 | Song Nam – Song Bac HPS | 49 | IPP |
| 6 | Tra Khuc 1 HPS | 36 | IPP |
| 7 | Song Tranh 4 HPS | 48 | IPP |
| 8 | La Ngau HPS | 46 | La Ngau hydropower joint-stock company |
| 9 | Minor HPSs | 300 |  |
|  | Projects expected to be commenced in 2015 | 384 |  |
| 1 | Song Lo 6 HPS | 44 | IPP |
| 2 | Song Tranh 5 HPS | 40 | IPP |
| 3 | Thanh Son HPS | 40 | IPP |
| 4 | Phu Tan 2 HPS | 60 | IPP |
| 5 | Minor HPSs | 200 |  |
|  | Projects expected to be commenced in 2016 | 355 |  |
| 1 | Thanh Son HPS | 37 | IPP |
| 2 | Ban Mong HPS | 60 | IPP |
| 3 | A Lin HPS | 63 | IPP |
| 4 | Dak My 3 HPS | 45 | IPP |
| 5 | Minor HPSs | 150 |  |
|  | Projects expected to be commenced in 2017 | 354 |  |
| 1 | Dak My 1 HPS | 54 | IPP |
| 2 | Minor HPSs | 300 |  |

APPENDIX IV:

LIST OF ELECTRICAL GRID PROJECTS EXPECTED TO BE BUILT OVER THE PERIOD FROM 2011 TO 2020 
(Enclosed with the Decision No. 1208/QĐ-TTg dated July 21, 2011 of the Prime Minister)

500kV SUBSTATIONS BUILT OVER THE PERIOD FROM 2011 TO 2020

| No. | Project name | Number of transformers x MVA |  |  | Number of transformers x MVA Number of transformers x MVA Capacity - MVA | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | Substation built over the period from 2011 to 2020Substation built over the period from 2011 to 2020Substation built over the period from 2011 to 2020Substation built over the period from 2011 to 2020Substation built over the period from 2011 to 2020Substation built over the period from 2011 to 2020 |  |  |  |  |  |
|  | Northern area |  |  |  | Northern areaNorthern areaNorthern area6750 |  |
| 1 | Nho Quan | 1 | x | 450 | 450 | Install transformer 2 |
| 2 | Ha Tinh | 1 | x | 450 | 450 | Install transformer 2 |
| 3 | Hiep Hoa | 2 | x | 900 | 1800 | Newly constructed |
| 4 | Pho Noi | 2 | x | 900 | 1800 | Newly constructed |
| 5 | Viet Tri | 1 | x | 450 | 450 | Newly constructed |
| 6 | Vung Ang | 2 | x | 450 | 900 | Connected to Vung Ang 1 |
| 7 | Quang Ninh | 1 | x | 450 | 450 | Install transformer 2 |
| 8 | Lai Chau | 1 | x | 450 | 450 | Be considered to build in order to gather the minor HPSs in the area |
|  | Central area |  |  |  | Central areaCentral areaCentral area900 |  |
| 1 | Thanh My | 2 | x | 450 | 900 | Newly constructed |
|  | Southern area |  |  |  | Southern areaSouthern areaSouthern area9450 |  |
| 1 | Phu Lam | 1 | x | 900 | 900 | Replace transformer 1 |
| 2 | Song May | 2 | x | 600 | 1200 | 2013 |
| 3 | Cau Bong | 2 | x | 900 | 1800 | Have 3 transformers |
| 4 | Duc Hoa | 1 | x | 900 | 900 |  |
| 5 | Tan Uyen | 1 | x | 900 | 900 | Rename Thu Duc Bac substation |
| 6 | Thot Not | 1 | x | 600 | 600 |  |
| 7 | My Tho | 1 | x | 900 | 900 |  |
| 8 | O Mon | 1 | x | 450 | 450 | Install transformer 2 |
| 9 | Long Phu | 1 | x | 450 | 450 | Shall be built if the progress of the 500kV transmission line Long Phu – O Mon is late compared with the progress of Long Phu TPPs 1, 2 |
| 10 | Duyen Hai | 1 | x | 450 | 450 |  |
| 11 | Vinh Tan | 2 | x | 450 | 900 | Connected to Vinh Tan 2 |
|  | Substation built over the period from 2016 to 2020Substation built over the period from 2016 to 2020Substation built over the period from 2016 to 2020Substation built over the period from 2016 to 2020Substation built over the period from 2016 to 2020Substation built over the period from 2016 to 2020 |  |  |  |  |  |
|  | Northern area |  |  |  | Northern areaNorthern areaNorthern area13100 |  |
| 1 | Thuong Tin | 2 | x | 900 | 1800 | Replace transformer |
| 2 | Nho Quan | 1 | x | 900 | 900 | Replace transformer 1 |
| 3 | Viet Tri | 1 | x | 900 | 900 | Install transformer 2 |
| 4 | Vinh Yen converter station |  |  |  | 2000 | Built in concurrence with the progress of importating power from China (3000 ~ 4000 MV ); details are in the investment project. |
| 5 | Dong Anh | 2 | x | 900 | 1800 | Newly constructed |
| 6 | Tay Ha Noi | 1 | x | 900 | 900 | Newly constructed |
| 7 | Long Bien | 2 | x | 900 | 1800 | Newly constructed |
| 8 | Thai Binh switching substation |  |  |  |  | Swithching substation (need preparation for installing 2 Transformers) |
| 9 | Nghi Son | 2 | x | 450 | 900 | Connected to Nghi Son 2 TPP |
| 10 | Bac Ninh | 2 | x | 600 | 1200 | Newly constructed (with 03 Transformers) |
| 11 | Hai Phong | 1 | x | 900 | 900 | Newly constructed |
|  | Central area |  |  |  | Central areaCentral areaCentral area2400 |  |
| 1 | Hue | 1 | x | 600 | 600 | Install transformer 1 |
| 2 | Doc Soi | 1 | x | 450 | 450 | Install transformer 2 |
| 3 | Da Nang | 1 | x | 900 | 900 | Replace transformer 1 |
| 4 | Van Phong | 1 | x | 450 | 450 | Connected to Van Phong |
|  | Southern area |  |  |  | Southern areaSouthern areaSouthern area11250 |  |
| 1 | Phu Lam | 1 | x | 900 | 900 | Replace transformer 2 |
| 2 | Nha Be | 2 | x | 900 | 1800 | Replace 2 Transformers |
| 3 | Tan Uyen | 1 | x | 900 | 900 | Install transformer 2 |
| 4 | My Phuoc | 1 | x | 900 | 900 | expected to be commenced in 2016 |
| 5 | Dong Nai |  |  |  |  | Switching substation (need preparation for installing 3 Transformers) |
| 6 | Cu Chi | 2 | x | 900 | 1800 |  |
| 7 | Binh Duong 1 | 1 | x | 900 | 900 | Connected to Ninh Thuan 1 NPP |
| 8 | Long Thanh | 2 | x | 900 | 1800 | Newly constructed |
| 9 | Duc Hoa | 1 | x | 900 | 900 | Install transformer 2 |
| 10 | My Tho | 1 | x | 900 | 900 | Install transformer 2 |
| 11 | Kien Luong TPP | 1 | x | 450 | 450 |  |

500kV TRANSMISSION LINES BUILT OVER THE PERIOD FROM 2011 TO 2020

| No. | Project name | Number of Circuits x km |  |  | Number of Circuits x kmNumber of Circuits x kmkm | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | Transmission lines built over the period from 2011 to 2020Transmission lines built over the period from 2011 to 2020Transmission lines built over the period from 2011 to 2020Transmission lines built over the period from 2011 to 2020Transmission lines built over the period from 2011 to 2020Transmission lines built over the period from 2011 to 2020 |  |  |  |  |  |
|  | Northern area |  |  |  | 1443 |  |
| 1 | Son La – Hiep Hoa | 2 | x | 286 | 572 | 2011 |
| 2 | Quang Ninh – Hiep Hoa | 2 | x | 140 | 280 |  |
| 3 | Quang Ninh - Mong Duong | 2 | x | 25 | 50 | 2013 |
| 4 | Pho Noi – turn at Quang Ninh – Thuong Tin | 4 | x | 10 | 40 |  |
| 5 | Vung Ang – turn at Ha Tinh – Da Nang | 4 | x | 18 | 72 |  |
| 6 | Son La - Lai Chau | 2 | x | 180 | 360 |  |
| 7 | Hiep Hoa - Dong Anh - Pho Noi | 1 | x | 69 | 69 |  |
| 8 | Increase in the capacity of the capacitors along the part Nho Quan – Ha Tinh |  |  |  |  | 2000A or above |
|  | Central area |  |  |  | 1074 |  |
| 1 | Pleiku – My Phuoc – Cau Bong | 2 | x | 437 | 874 |  |
| 2 | HatXan - Pleiku | 2 | x | 100 | 200 |  |
| 3 | Increase in the capacity of the capacitors along the part Ha Tinh – Da Nang |  |  |  |  | 2000A or above |
|  | Southern area |  |  |  | 1316 |  |
| 1 | Song May – Tan Dinh | 2 | x | 41 | 82 | 2012 |
| 2 | Phu My – Song May | 2 | x | 66 | 132 | 2012 |
| 3 | Vinh Tan – Song May | 2 | x | 235 | 470 | Connected to Vinh Tan 2 TPP |
| 4 | Splitting point - Cau Bong | 4 | x | 1 | 4 | Transitted 2 circuits of current 500kv lines |
| 5 | Splitting point - Duc Hoa | 4 | x | 8 | 32 | Transitted 2 circuits of current 500kv lines |
| 6 | Song May – Tan Uyen | 2 | x | 22 | 44 | Using big cable |
| 7 | My Tho - Duc Hoa | 2 | x | 60 | 120 | Connected to Duyen Hai & Long Phu TPPs |
| 8 | Duyen Hai – My Tho TPP | 2 | x | 113 | 226 | Connected to Duyen Hai 1 TPP |
| 9 | Long Phu – O Mon TPP | 2 | x | 84 | 168 | Connected to Long Phu 1 TPP |
| 10 | O Mon – Thot Not | 2 | x | 16 | 32 |  |
| 11 | Splitting point - My Tho | 4 | x | 1 | 4 | Splitting points on the 2 transmission lines O Mon – Phu Lam and Nha Be |
| 12 | Increase in the capacity of the capacitors along the part PleiKu – Dak Nong – Phu Lam and the part PleiKu – Di Linh – Tan Dinh |  |  |  |  | 2000A or above |
| 13 | Phu My 4 – Phu My | 2 | x | 1 | 2 | Replace with bigger cable |
|  | Substation built over the period from 2016 to 2020Substation built over the period from 2016 to 2020Substation built over the period from 2016 to 2020Substation built over the period from 2016 to 2020Substation built over the period from 2016 to 2020Substation built over the period from 2016 to 2020 |  |  |  |  |  |
|  | Northern area |  |  |  | 2015 |  |
| 1 | Bac Ninh – splitting point Dong Anh - Pho Noi | 2 | x | 5 | 10 |  |
| 2 | Splitting point Bac Ninh - Quang Ninh – Hiep Hoa | 2 | x | 32 | 64 |  |
| 3 | Tay Ha Noi - Thuong Tin | 2 | x | 24 | 48 |  |
| 4 | 500kV Transmission line for importing power from China | 350 |  |  | 350350350 | DC shall be considered to be used |
| 5 | Vinh Yen – Tay Ha Noi converter station | 2 | x | 70 | 140 | In case using DC to import power from China. |
| 6 | Tay Ha Noi – splitting point Viet Tri – Hiep Hoa (Vinh Yen) | 2 | x | 60 | 120 | If the 500kV power has not been imported from China. |
| 7 | Dong Phu Yen – Tay Ha Noi | 2 | x | 110 | 220 |  |
| 8 | Dong Phu Yen – splitting point Son La – Viet Tri | 4 | x | 20 | 80 | Connected to Dong Phu Yen HAPP |
| 9 | Nho Quan - Thuong Tin | 1 | x | 75 | 75 | For circuit 2 |
| 10 | Long Bien – splitting point Thuong Tin – Pho Noi | 2 | x | 5 | 10 |  |
| 11 | Thang Long TPP – splitting point Quang Ninh – Pho Noi | 2 | x | 5 | 10 | Connected to Thang Long TPP |
| 12 | Thai Binh – Pho Noi switching substation | 2 | x | 60 | 120 |  |
| 13 | Thai Binh - Nam Dinh TPP switching substation | 2 | x | 72 | 144 |  |
| 14 | Thai Binh – Hai Phong switching substation | 2 | x | 40 | 80 |  |
| 15 | Nghi Son – splitting point Nho Quan – Ha Tinh | 4 | x | 50 | 200 |  |
| 16 | Quang Trach TPP – Vung Ang | 2 | x | 25 | 50 |  |
| 17 | Nam Dinh TPP – Thanh Hoa | 2 | x | 72 | 144 |  |
| 18 | Quynh Lap TPP – Thanh Hoa | 2 | x | 75 | 150 |  |
|  | Central area |  |  |  | 630 |  |
| 1 | Van Phong TPP – Ninh Thuan NPP 1 – Vinh Tan TPP | 2 | x | 195 | 390 | Connected to Van Phong 1 TPP |
| 2 | Hue – splitting point Ha Tinh – Da Nang | 4 | x | 5 | 20 |  |
| 3 | Bac Ai HAPP - splitting point Van Phong – Ninh Thuan NPP 1 | 4 | x | 35 | 140 |  |
| 4 | Quang Tri TPP – splitting point Ha Tinh – Da Nang | 4 | x | 20 | 80 |  |
|  | Southern area |  |  |  | 1894 |  |
| 1 | Vinh Tan Electricity Center – Song May | 2 | x | 235 | 470 | Circuits 3, 4 – Connected to the project finished sooner (Vinh Tan 3 or Van Phong 1) |
| 2 | Ninh Thuan 1 NPP – Binh Duong 1 | 2 | x | 250 | 500 | Connected to Ninh Thuan 1 NPP |
| 3 | Long Thanh – splitting point Phu My – Song May | 2 | x | 16 | 32 |  |
| 4 | Dong Nai – splitting point Phu My – Song May | 4 | x | 18 | 72 |  |
| 5 | Son My electricity center – Dong Nai switching substation | 2 | x | 53 | 106 | Connected to Son My 1 TPP |
| 6 | My Phuoc – Cu Chi | 2 | x | 25 | 50 |  |
| 7 | Kien Luong TPP - Thot Not | 2 | x | 107 | 214 | Connected to Kien Luong TPP |
| 8 | Thot Not - Duc Hoa | 2 | x | 145 | 290 |  |
| 9 | My Phuoc – splitting point Pleiku – Cau Bong | 4 | x | 5 | 20 |  |
| 10 | Splitting point Song May – Tan Dinh – Binh Duong 1 | 4 | x | 5 | 20 |  |
| 11 | Song Hau TPP – My Tho | 2 | x | 60 | 120 | be built in case the Song Hau 1 TPP project is commenced sooner than expected |

220kV SUBSTATIONS BUILT OVER THE PERIOD FROM 2011 TO 2020

| No. | Project name | Number of transformers x MVA |  |  |  | Number of transformers x MVA Number of transformers x MVA Capacity - MVA |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Substations built over the period from 2011 to 2015Substations built over the period from 2011 to 2015Substations built over the period from 2011 to 2015Substations built over the period from 2011 to 2015Substations built over the period from 2011 to 2015Substations built over the period from 2011 to 2015 |  |  |  |  |  |  |  |
|  | Northern area |  |  |  |  | Northern areaNorthern areaNorthern area15438 |  |  |
| 1 | Thanh Cong | 2 | x | 250 | 500 |  |  |  |
| 2 | Viet Tri | 2 | x | 250 | 500 |  |  |  |
| 3 | Xuan Mai | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 4 | Dong Hoa | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 5 | Vat Cach | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 6 | Dinh Vu | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 7 | Hai Phong TPP | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 8 | Hai Duong 1 | 2 | x | 125+250 | 375 |  | Install transformer 1, 2 |  |
| 9 | Pho Noi | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 10 | Phu Ly | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 11 | Nam Dinh | 1 | x | 250 | 250 |  | Replace the MBA |  |
| 12 | Thai Binh | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 13 | Hoa Lu | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 14 | Lao Cai | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 15 | Tuyen Quan HPS | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 16 | Yen Bai | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 17 | Viet Tri | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 18 | Vinh Yen | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 19 | Bac Ninh | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 20 | Trang Bach | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 21 | Hoanh Bo | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 22 | Uong Bi | 1 | x | 250 | 250 |  | Replace transformer |  |
| 23 | Yen Hung | 1 | x | 250 | 250 |  | Newly constructed |  |
| 24 | Khe Than | 1 | x | 63 | 63 |  | Support coal consumption |  |
| 25 | Quang Ninh TPP | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 26 | Son La commune | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 27 | Hoa Binh | 1 | x | 125 | 250 |  | Replace transformer |  |
| 28 | Thanh Hoa | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 29 | Nghi Son | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 30 | Vinh | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 31 | Do Luong | 2 | x | 125 | 250 |  | Newly constructed |  |
| 32 | Ha Tinh | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 33 | An Duong (Tay Ho) | 1 | x | 250 | 250 |  | Newly constructed |  |
| 34 | Dong Anh | 1 | x | 250 | 250 |  | Newly constructed |  |
| 35 | Long Bien | 1 | x | 250 | 250 |  | Newly constructed |  |
| 36 | Tay Ha Noi | 1 | x | 250 | 250 |  | Newly constructed – install transformer 1 |  |
| 37 | Son Tay | 1 | x | 250 | 250 |  | Newly constructed |  |
| 38 | Thuong Tin | 1 | x | 250 | 250 |  | Newly constructed – install transformer 1 |  |
| 39 | Thuy Nguyen | 1 | x | 250 | 250 |  | Newly constructed |  |
| 40 | Duong Kinh | 1 | x | 250 | 250 |  | Newly constructed |  |
| 41 | Hai Duong 2 | 1 | x | 250 | 250 |  | Newly constructed |  |
| 42 | Hai Duong TPP | 1 | x | 250 | 250 |  | Newly constructed |  |
| 43 | Thanh Nghi | 1 | x | 250 | 250 |  | Newly constructed |  |
| 44 | Truc Ninh | 1 | x | 250 | 250 |  | Newly constructed |  |
| 45 | Thai Binh TPP | 1 | x | 250 | 250 |  | Connected to Thai Binh TPP |  |
| 46 | Ninh Binh 2 | 1 | x | 125 | 125 |  | Newly constructed |  |
| 47 | Nho Quan | 1 | x | 125 | 125 |  | In the 500kV subsubstation |  |
| 48 | Ha Giang | 1 | x | 125 | 125 |  | Install transformers in the switching substation |  |
| 49 | Cao Bang | 1 | x | 125 | 125 |  | Newly constructed |  |
| 50 | Bao Thang | 1 | x | 250 | 250 |  | Newly constructed |  |
| 51 | Than Uyen | 1 | x | 125 | 125 |  | Newly constructed |  |
| 52 | Bac Kan |  |  |  |  |  | Switching substation |  |
| 53 | Lang Son | 1 | x | 125 | 125 |  | Newly constructed |  |
| 54 | Tuyen Quang | 1 | x | 125 | 125 |  | Newly constructed |  |
| 55 | Luu Xa | 1 | x | 250 | 250 |  | Newly constructed |  |
| 56 | Yen Bai | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 57 | Tuan Giao | 2 | x | 250 | 500 |  | For combining minor HPSs in the area |  |
| 58 | Phu Tho | 1 | x | 250 | 250 |  | Newly constructed |  |
| 59 | Ba Thien | 1 | x | 250 | 250 |  | Newly constructed |  |
| 60 | Quang Chau | 1 | x | 125 | 125 |  | Newly constructed |  |
| 61 | Tien Son (Bac Ninh -2) | 1 | x | 250 | 250 |  | Newly constructed |  |
| 62 | Yen Phong (Bac Ninh -3) | 1 | x | 250 | 250 |  | Newly constructed |  |
| 63 | Hai Ha industrial zone | 1 | x | 250 | 250 |  | Newly constructed |  |
| 64 | Yen Thuy | 1 | x | 125 | 125 |  | Newly constructed |  |
| 65 | Bim Son | 2 | x | 125+250 | 375 |  | Newly constructed. Install transformer 1 in 2012 |  |
| 66 | Quynh Luu | 1 | x | 250 | 250 |  | Newly constructed |  |
| 67 | Thach Khe | 2 | x | 125 | 250 |  | Newly constructed |  |
| 68 | Vung Ang | 1 | x | 250 | 250 |  | Newly constructed |  |
|  | Central area |  |  |  |  | Central areaCentral areaCentral area3925 |  |  |
| 1 | Thanh My | 1 | x | 125 | 125 |  | Establishing connection to the HPS |  |
| 2 | Doc Soi | 1 | x | 125 | 125 |  | Replace 63MVA transformer |  |
| 3 | Dung Quat 2 | 2 | x | 125 | 250 |  |  |  |
| 4 | Son Ha | 2 | x | 150 | 300 |  |  |  |
| 5 | Tuy Hoa | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 6 | Hue | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 7 | Chan May | 1 | x | 250 | 250 |  | Install transformer 1 |  |
| 8 | Phong Dien | 1 | x | 125 | 125 |  | Install transformer 1 |  |
| 9 | Hoa Khanh | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 10 | District 3 (Ngu Hanh Son) | 1 | x | 125 | 125 |  | Install transformer 1 |  |
| 11 | Da Nang | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 12 | Tam Ky | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 13 | Tam Hiep | 1 | x | 125 | 125 |  | Install transformer 1 |  |
| 14 | Dung Quat | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 15 | An Nhon | 1 | x | 250 | 250 |  | Install transformer 1 |  |
| 16 | Nha Trang | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 17 | Van Phong | 1 | x | 250 | 250 |  | Install transformer 1 |  |
| 18 | Kon Tum | 1 | x | 125 | 125 |  | Install transformer 1 |  |
| 19 | PleiKu | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 20 | Ba Don | 1 | x | 125 | 125 |  |  |  |
| 21 | Buon Kuop | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 22 | Song Tranh 2 | 1 | x | 125 | 125 |  | Newly constructed |  |
|  | Southern area |  |  |  |  | Southern areaSouthern areaSouthern area16.500 |  |  |
| 1 | Bao Loc | 1 | x | 125 | 125 |  | Replace 63MVA transformer |  |
| 2 | Duc Trong | 1 | x | 125 | 125 |  |  |  |
| 3 | Thap Cham | 1 | x | 125 | 125 |  |  |  |
| 4 | Phan Thiet | 2 | x | 125+250 | 375 |  | Replace transformer 1 + install transformer 2 |  |
| 5 | Ham Tan | 1 | x | 250 | 250 |  |  |  |
| 6 | Cat Lai | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 7 | Nam Sai Gon | 2 | x | 250 | 500 |  |  |  |
| 8 | Hiep Binh Phuoc | 2 | x | 250 | 500 |  |  |  |
| 9 | Binh Tan | 2 | x | 250 | 500 |  |  |  |
| 10 | Cu Chi | 2 | x | 250 | 500 |  |  |  |
| 11 | Cau Bong | 2 | x | 250 | 500 |  |  |  |
| 12 | Nhon Trach city | 2 | x | 250 | 500 |  |  |  |
| 13 | Tan Uyen | 2 | x | 250 | 500 |  |  |  |
| 14 | Tan Cang | 2 | x | 250 | 500 |  |  |  |
| 15 | District 8 | 2 | x | 250 | 500 |  |  |  |
| 16 | Binh Long | 2 | x | 125 | 500 |  |  |  |
| 17 | Tay Ninh | 2 | x | 250 | 500 |  |  |  |
| 18 | My Phuoc | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 19 | Thuan An | 2 | x | 250 | 500 |  |  |  |
| 20 | Uyen Hung | 2 | x | 250 | 500 |  |  |  |
| 21 | Ben Cat | 2 | x | 250 | 500 |  |  |  |
| 22 | Tan Dinh 2 | 1 | x | 250 | 250 |  |  |  |
| 23 | Song May | 2 | x | 250 | 500 |  |  |  |
| 24 | Xuan Loc | 2 | x | 250 | 500 |  |  |  |
| 25 | Vung Tau | 2 | x | 250 | 500 |  |  |  |
| 26 | Phu My 2 industrial zone | 2 | x | 250 | 500 |  |  |  |
| 27 | My Xuan | 2 | x | 250 | 500 |  |  |  |
| 28 | Chau Duc | 1 | x | 250 | 250 |  |  |  |
| 29 | Long An | 2 | x | 250 | 500 |  | Replace 02 125MVA transformers |  |
| 30 | Ben Luc | 1 | x | 250 | 250 |  |  |  |
| 31 | Duc Hoa | 2 | x | 250 | 500 |  |  |  |
| 32 | Can Duoc | 1 | x | 250 | 250 |  |  |  |
| 33 | Cao Lanh | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 34 | Sa Dec Industrial Zone | 1 | x | 250 | 250 |  |  |  |
| 35 | Chau Doc | 2 | x | 250 | 500 |  | Replace 02 transformers |  |
| 36 | Long Xuyen 2 | 1 | x | 250 | 250 |  |  |  |
| 37 | My Tho | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 38 | Cai Lay | 1 | x | 250 | 250 |  | Replace 01 transformer |  |
| 39 | Vinh Long 2 | 1 | x | 250 | 250 |  | Replace 01 transformer |  |
| 40 | Ben Tre | 1 | x | 250 | 250 |  | Replace 01 transformer |  |
| 41 | Thot Not | 1 | x | 250 | 250 |  | Replace 01 transformer |  |
| 42 | Phung Hiep | 2 | x | 125 | 250 |  |  |  |
| 43 | Tra Vinh | 2 | x | 125 | 250 |  |  |  |
| 44 | Soc Trang | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 45 | Ca Mau | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 46 | Vinh Long | 2 | x | 125+250 | 375 |  |  |  |
| 47 | Bac Lieu | 1 | x | 125 | 125 |  |  |  |
|  | Substations built over the period from 2016 to 2020Substations built over the period from 2016 to 2020Substations built over the period from 2016 to 2020Substations built over the period from 2016 to 2020Substations built over the period from 2016 to 2020Substations built over the period from 2016 to 2020 |  |  |  |  |  |  |  |
|  | Northern area |  |  |  |  | Northern areaNorthern areaNorthern area18063 |  |  |
| 1 | An Duong (Tay Ho) | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 2 | Dong Anh | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 3 | Xuan Mai | 2 | x | 250 | 500 |  | Replace transformer |  |
| 4 | Son Tay | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 5 | Hoa Lac | 1 | x | 250 | 250 |  | Install transformer 1 |  |
| 6 | Hoa Binh | 1 | x | 125 | 125 |  | Replace transformer 2 |  |
| 7 | Tay Ha Noi | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 8 | Long Bien | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 9 | Thuong Tin | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 10 | Kim Dong | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 11 | Quang Ninh TPP | 1 | x | 250 | 250 |  | Replace transformer MBA1 |  |
| 12 | Hai Phong TPP | 1 | x | 250 | 250 |  | Replace transformer |  |
| 13 | Vat Cach | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 14 | Thuy Nguyen | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 15 | Duong Kinh | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 16 | Hai Duong 2 | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 17 | Hai Duong TPP | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 18 | Phu Ly | 1 | x | 250 | 250 |  | Replace transformer 1 + install transformer 2 |  |
| 19 | Truc Ninh | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 20 | Thai Binh | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 21 | Thai Binh TPP | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 22 | Hoa Lu | 1 | x | 250 | 250 |  | Replace transformer |  |
| 23 | Ninh Binh 2 | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 24 | Quy Nhon | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 25 | Ha Giang | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 26 | Cao Bang | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 27 | Than Uyen | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 28 | Bac Kan | 1 | x | 125 | 125 |  | Install transformer 1 |  |
| 29 | Tuyen Quang | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 30 | Tuyen Quang HPS | 1 | x | 125 | 125 |  | Replace 63MVA transformer |  |
| 31 | Yen Bai | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 32 | Luu Xa | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 33 | Viet Tri | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 34 | Vinh Yen | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 35 | Ba Thien | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 36 | Bac Giang | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 37 | Quang Chau | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 38 | Tien Son (Bac Ninh 2) | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 39 | Yen Phong (Bac Ninh -3) | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 40 | Hai Duong 3 switching substation |  | x |  |  |  | For separating the point Pha Lai in order to deduce short-circuit |  |
| 41 | Trang Bach | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 42 | Thanh Hoa | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 43 | Nghi Son | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 44 | Vinh | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 45 | Son La commune | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 46 | Yen Thuy | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 47 | Bim Son | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 48 | Quynh Luu | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 49 | Ha Tinh | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 50 | Vung Ang | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 51 | Dong Anh 2 | 2 | x | 250 | 500 |  | Newly constructed |  |
| 52 | Van Dien | 2 | x | 250 | 500 |  | Newly constructed |  |
| 53 | Chuong My | 1 | x | 250 | 250 |  | Newly constructed |  |
| 54 | Ung Hoa | 2 | x | 250 | 500 |  | Newly constructed |  |
| 55 | Me Linh | 1 | x | 250 | 250 |  | Newly constructed |  |
| 56 | An Lao | 2 | x | 250 | 500 |  | Newly constructed |  |
| 57 | Bac Song Cam | 2 | x | 250 | 500 |  | Newly constructed |  |
| 58 | Gia Loc | 1 | x | 250 | 250 |  | Newly constructed |  |
| 59 | Pho Cao | 1 | x | 250 | 250 |  | Newly constructed |  |
| 60 | Yen My | 1 | x | 250 | 250 |  | Newly constructed |  |
| 61 | Ly Nhan | 1 | x | 125 | 125 |  | Newly constructed |  |
| 62 | Vu Thu | 1 | x | 250 | 250 |  | Newly constructed |  |
| 63 | Quynh Phu | 1 | x | 250 | 250 |  | Newly constructed |  |
| 64 | Nghia Lo | 1 | x | 125 | 125 |  | Newly constructed |  |
| 65 | Phu Binh | 1 | x | 250 | 250 |  | Newly constructed |  |
| 66 | Vinh Tuong | 1 | x | 250 | 250 |  | Newly constructed |  |
| 67 | Tan Yen | 1 | x | 250 | 250 |  | Newly constructed |  |
| 68 | Thua | 2 | x | 250 | 500 |  | Newly constructed |  |
| 69 | Yen Hung | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 70 | Lach Huyen Steel | 1 | x | 125 | 125 |  | Newly constructed |  |
| 71 | Lai Chau | 1 | x | 125 | 125 |  | Newly constructed |  |
| 72 | Khe Than | 1 | x | 63 | 63 |  | Install transformer 2 |  |
| 73 | Nong Cong | 1 | x | 125 | 125 |  | Newly constructed |  |
| 74 | Ngoc Lac | 1 | x | 125 | 125 |  | Newly constructed |  |
| 75 | Nam Cam | 1 | x | 250 | 250 |  | Newly constructed |  |
|  | Central area |  |  |  |  | Central areaCentral areaCentral area3750 |  |  |
| 1 | Dong Ha | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 2 | Lao Bao | 1 | x | 125 | 125 |  | Install transformer 1 |  |
| 3 | Hue | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 4 | Phong Dien | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 5 | Cau Hai | 1 | x | 125 | 125 |  | Install transformer 1 |  |
| 6 | Da Nang | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 7 | Lien Chieu | 1 | x | 250 | 250 |  | Install transformer 1 |  |
| 8 | An Don (Tien Sa) | 1 | x | 250 | 250 |  | Install transformer 1 |  |
| 9 | Hoi An | 1 | x | 250 | 250 |  | Install transformer 1 |  |
| 10 | Tam Hiep | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 11 | Quang Ngai | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 12 | Hoai Nhon | 1 | x | 125 | 125 |  | Install transformer 1 |  |
| 13 | Tuy Hoa | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 14 | Quy Nhon | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 15 | Song Cau | 1 | x | 125 | 125 |  | Install transformer 1 |  |
| 16 | Cam Ranh | 1 | x | 250 | 250 |  | Install transformer 1 |  |
| 17 | Ninh Hoa | 1 | x | 250 | 250 |  | Install transformer 1 |  |
| 18 | Bo Y | 1 | x | 125 | 125 |  | Install transformer 1 |  |
| 19 | Dak Nong | 1 | x | 125 | 125 |  |  |  |
| 20 | Chu Se | 1 | x | 125 | 125 |  | Install transformer 1 |  |
|  | Southern area |  |  |  |  | Southern area Southern area Southern area17250 |  |  |
| 1 | Tan Rai | 1 | x | 250 | 250 |  |  |  |
| 2 | Duc Trong | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 3 | Thap Cham | 1 | x | 125 | 125 |  | Install transformer 2 |  |
| 4 | Phan Ri | 1 | x | 250 | 250 |  |  |  |
| 5 | Ham Tan | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 6 | Tan Son Nhat | 2 | x | 250 | 500 |  |  |  |
| 7 | Dam Sen | 2 | x | 250 | 500 |  |  |  |
| 8 | Nam Hiep Phuoc | 2 | x | 250 | 500 |  |  |  |
| 9 | District 9 | 1 | x | 250 | 250 |  | Newly constructed |  |
| 10 | Binh Chanh | 2 | x | 250 | 500 |  |  |  |
| 11 | Thu Thiem | 2 | x | 250 | 500 |  |  |  |
| 12 | Vinh Loc | 2 | x | 250 | 500 |  | For replacing temporary substation with full-fledged substation |  |
| 13 | Binh Long | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 14 | Phuoc Long | 1 | x | 250 | 250 |  | Newly constructed |  |
| 15 | Tay Ninh 2 | 2 | x | 250 | 500 |  |  |  |
| 16 | Binh Hoa | 1 | x | 250 | 250 |  | Install transformer 3 |  |
| 17 | Thuan An | 1 | x | 250 | 250 |  | Install transformer 3 |  |
| 18 | Tan Dinh 2 | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 19 | Lai Uyen | 2 | x | 250 | 500 |  |  |  |
| 20 | Tan Uyen | 2 | x | 250 | 500 |  |  |  |
| 21 | Binh Duong 1 | 2 | x | 250 | 500 |  | Etablishing connections in the 500kV substation |  |
| 22 | Long Thanh | 1 | x | 250 | 250 |  | Install transformer 3 |  |
| 23 | Phu Hoa Dong | 2 | x | 250 | 500 |  | Newly constructed |  |
| 24 | Tay Bac Cu Chi | 2 | x | 250 | 500 |  |  |  |
| 25 | Song May | 1 | x | 250 | 250 |  | Install transformer 3 |  |
| 26 | An Phuoc | 2 | x | 250 | 500 |  |  |  |
| 27 | Nhon Trach city | 1 | x | 250 | 250 |  | Install transformer 3 |  |
| 28 | Tam Phuoc | 2 | x | 250 | 500 |  |  |  |
| 29 | Phu My 2 industrial zone | 1 | x | 250 | 250 |  | Install transformer 3 |  |
| 30 | Dat Do | 1 | x | 250 | 250 |  | Install transformer 1 |  |
| 31 | Phu My 3 industrial zone | 2 | x | 250 | 500 |  |  |  |
| 32 | Ben Luc | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 33 | Tan An | 1 | x | 250 | 250 |  |  |  |
| 34 | Cao Lanh | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 35 | Lap Vo | 2 | x | 250 | 500 |  |  |  |
| 36 | Long Xuyen 2 | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 37 | Chau Thanh | 1 | x | 250 | 250 |  |  |  |
| 38 | Cai Lay | 1 | x | 250 | 250 |  | Replace 01 transformer |  |
| 39 | My Tho | 1 | x | 250 | 250 |  | Replace transformer 2 |  |
| 40 | Go Cong | 2 | x | 250 | 500 |  | Newly constructed |  |
| 41 | Vinh Long 2 | 1 | x | 250 | 250 |  | Replace 01 transformer |  |
| 42 | Vinh Long 3 | 1 | x | 125 | 125 |  |  |  |
| 43 | Ben Tre | 1 | x | 250 | 250 |  | Replace 01 transformer |  |
| 44 | Sa Dec Industrial Zone | 1 | x | 250 | 250 |  | Install transformer 2 |  |
| 45 | Mo Cay | 1 | x | 125 | 125 |  |  |  |
| 46 | Kien Binh | 2 | x | 250 | 500 |  | Replace 02 transformer |  |
| 47 | Go Quao | 1 | x | 125 | 125 |  |  |  |
| 48 | Thot Not | 1 | x | 250 | 250 |  | Replace 01 transformer |  |
| 49 | Ninh Kieu | 1 | x | 125 | 125 |  |  |  |
| 50 | Vi Thanh | 1 | x | 125 | 125 |  |  |  |
| 51 | Rach Gia | 1 | x | 250 | 250 |  | Replace transformer 1 |  |
| 52 | Duyen Hai | 1 | x | 250 | 250 |  |  |  |
| 53 | My Tu | 1 | x | 125 | 125 |  |  |  |
| 54 | Long Phu TPP | 1 | x | 125 | 125 |  |  |  |
| 55 | Gia Rai | 1 | x | 125 | 125 |  |  |  |
| 56 | Ngoc Hien | 2 | x | 125 | 250 |  |  |  |

220kV TRANSMISSION LINES EXPECTED TO BE BUILT OVER THE PERIOD FROM 2011 TO 2020

| No | Project name | Number of Circuits x km |  |  | Number of Circuits x kmNumber of Circuits x kmkm |  |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tranmission lines built over the period from 2011 to 2020 |  |  |  |  |  | Tranmission lines built over the period from 2011 to 2020Tranmission lines built over the period from 2011 to 2020Tranmission lines built over the period from 2011 to 2020Tranmission lines built over the period from 2011 to 2020 |  |  |
|  | Northern area |  |  |  |  | 3864 |  |  |  |
| 1 | Tay Ho – splitting point Chem – Van Tri | 2 | x | 7 |  | 14 |  |  |  |
| 2 | Soc Son – Van Tri | 2 | x | 22 |  | 44 |  |  |  |
| 3 | Dong Anh – Long Bien | 2 | x | 17 |  | 34 |  |  |  |
| 4 | Dong Anh – Hiep Hoa | 2 | x | 24 |  | 48 |  |  |  |
| 5 | Splitting point Yen Phong (Bac Ninh 3) – Dong Anh – Hiep Hoa | 2 | x | 17 |  | 34 |  |  |  |
| 6 | Long Bien – Tien Son (Bac Ninh 2) | 2 | x | 20 |  | 40 |  |  |  |
| 7 | Tan Ha Noi – splitting point Ha Dong - Chem | 4 | x | 15 |  | 60 |  |  | Connected to 220kV Tay Ha Noi Substation |
| 8 | Tay Ha Noi – Hoa Binh | 2 | x | 46 |  | 92 |  |  |  |
| 9 | Son Tay – splitting point Hoa Binh – Viet Tri | 2 | x | 3 |  | 6 |  |  |  |
| 10 | Duong Kinh – splitting point Dong Hoa – Dinh Vu | 4 | x | 3 |  | 12 |  |  |  |
| 11 | Thuy Nguyen – splitting point Hai Phong – Vat Cach | 4 | x | 1 |  | 4 |  |  |  |
| 12 | Dong Hoa - Thai Binh | 1 | x | 54 |  | 54 |  |  | For Circuit 2 |
| 13 | Hai Duong 2 – splitting point Dong Hoa – Pha Lai | 2 | x | 2,5 |  | 5 |  |  |  |
| 14 | Hai Duong 2 – Mao Khe TPP | 2 | x | 22 |  | 44 |  |  |  |
| 15 | Hai Duong TPP – Pho Noi 500kV | 2 | x | 60 |  | 120 |  |  | Connected to Hai Duong TPP |
| 16 | Hai Duong TPP – splitting point Pha Lai – Hai Duong 2 | 4 | x | 2 |  | 8 |  |  | Connected to Hai Duong TPP |
| 17 | Kim Dong - Thai Binh | 2 | x | 70 |  | 140 |  |  |  |
| 18 | Thai BInh - Thai Binh TPP | 2 | x | 25 |  | 50 |  |  |  |
| 19 | Connection to Phoi Noi 500kV – Pho Noi – Pha Lai | 4 | x | 5 |  | 20 |  |  |  |
| 20 | Pho Noi 500kV – Tien Son (Bac Ninh 2) | 2 | x | 30 |  | 60 |  |  |  |
| 21 | Truc Ninh – splitting point Nam Dinh – Ninh Binh | 2 | x | 18 |  | 36 |  |  |  |
| 22 | Truc Ninh - Thai Binh TPP | 2 | x | 45 |  | 90 |  |  |  |
| 23 | Nho Quan – Phu Ly | 2 | x | 38,5 |  | 77 |  |  | Existing line shall be used |
| 24 | Tuan Giao – Son La | 2 | x | 90 |  | 180 |  |  |  |
| 25 | Cao Bang – Bac Kan | 2 | x | 78 |  | 156 |  |  |  |
| 26 | Cao Bang – Nho Que HPS | 2 | x | 98 |  | 196 |  |  |  |
| 27 | Bac Kan – splitting point Thai Nguyen – Tuyen Quang HPS | 2 | x | 20 |  | 40 |  |  |  |
| 28 | Lao Cai – Bao Thang | 1 | x | 20 |  | 20 |  |  |  |
| 29 | Bao Thang – Bac Ha HPS | 1 | x | 30 |  | 30 |  |  |  |
| 30 | Bao Thang – Yen Bai | 2 | x | 125 |  | 250 |  |  |  |
| 31 | Than Uyen – Ban Chat | 2 | x | 26 |  | 52 |  |  |  |
| 32 | Lang Son – Bac Giang | 2 | x | 95 |  | 190 |  |  |  |
| 33 | Phu Tho – splitting point Viet Tri – Yen Bai | 2 | x | 7 |  | 14 |  |  |  |
| 34 | Yen Son HPS – splitting point Tuyen Quang HPS – Tuyen Quang | 2 | x | 8 |  | 16 |  |  |  |
| 35 | Luu Xa – splitting point Soc Son – Thai Nguyen | 2 | x | 2 |  | 4 |  |  |  |
| 36 | Viet Tri – Viet Tri 500kV substation | 2 | x | 2 |  | 4 |  |  |  |
| 37 | Connection – Viet Tri 500kV substation | 4 | x | 2 |  | 8 |  |  |  |
| 38 | Soc Son – Ba Thien | 2 | x | 30 |  | 60 |  |  |  |
| 39 | Quang Chau – splitting point Pha Lai – Soc Son | 2 | x | 3 |  | 6 |  |  |  |
| 40 | Establishing connection to Hiep Hoa 500kV substation | 8 | x | 3 |  | 24 |  |  |  |
| 41 | Khe Than – splitting point Son Dong – Trang Bach | 2 | x | 2 |  | 4 |  |  |  |
| 42 | Yen Hung – splitting point Uong Bi – Trang Bach | 2 | x | 15 |  | 30 |  |  |  |
| 43 | Mao Khe TPP – splitting point Pha Lai – Trang Bach | 4 | x | 3 |  | 12 |  |  |  |
| 44 | Hai Ha – Cam Pha | 2 | x | 85 |  | 170 |  |  | For circuit 1 |
| 45 | Huoi Quang – Son La | 2 | x | 20 |  | 40 |  |  |  |
| 46 | Huoi Quang – Ban Chat | 2 | x | 22 |  | 44 |  |  |  |
| 47 | Nam Chien – Son La | 2 | x | 18 |  | 36 |  |  |  |
| 48 | Yen Thuy – splitting pointed Hoa Binh – Nho Quan | 2 | x | 1 |  | 2 |  |  |  |
| 49 | Nghi Son – Nghi Son TPP | 2 | x | 15 |  | 30 |  |  |  |
| 50 | Nghi Son – Thanh Hoa | 2 | x | 70 |  | 140 |  |  | Under construction |
| 51 | Nam Sum 1 TPP – Hua Na HPS | 2 | x | 35 |  | 70 |  |  |  |
| 52 | Hua Na HPS – Bim Son | 1 | x | 80 |  | 80 |  |  | Connected to circuit 2 of the line Nam Sum 1 – Hua Na |
| 53 | Bim Son – splitting point Ninh Binh – Thanh Hoa | 2 | x | 3 |  | 6 |  |  |  |
| 54 | Quynh Luu – splitting point Nghi Son - Vinh | 2 | x | 2 |  | 4 |  |  |  |
| 55 | Nam Mo – Ban La | 2 | x | 120 |  | 240 |  |  | Must ensure sufficient transmission for the HPSs in the area |
| 56 | Establishing connection to Cong Thanh TPP | 4 | x | 50 |  | 200 |  |  |  |
| 57 | Ha Tinh – Thach Khe | 2 | x | 20 |  | 40 |  |  |  |
| 58 | Ha Tinh – Vung Ang | 2 | x | 70 |  | 140 |  |  |  |
| 59 | Vung Ang – Dong Hoi | 2 | x | 90 |  | 180 |  |  |  |
| 60 | Thuong Tin – Kim Dong | 2 | x | 27 |  | 54 |  |  |  |
|  | Central area |  |  |  |  | 2867 |  |  |  |
| 1 | Dong Hoi – Dong Ha | 2 | x | 108 |  | 216 |  |  | Upgrade to double-circuit. Existing lines shall be considered to be used |
| 2 | Dong Ha - Hue | 2 | x | 68 |  | 136 |  |  | Ugrade to double-circuit. Existing lines shall be considered to be used |
| 3 | A Luoi HPS – splitting point Dong Ha - Hue | 2 | x | 28 |  | 56 |  |  | Connected to A Luoi HPS |
| 4 | Phong Dien – splitting point Hue – Hoa Khanh | 4 | x | 5 |  | 20 |  |  |  |
| 5 | Chan May – splitting point Hue – Hoa Khanh | 4 | x | 8 |  | 32 |  |  | Using 4-circuit towers |
| 6 | Hue - Hoa Khanh | 2 | x | 110 |  | 220 |  |  | Build new line of split-phase double circuit or upgrade existing line to double circuit line |
| 7 | SeKaman 3 - Thanh My | 2 | x | 120 |  | 240 |  |  |  |
| 8 | Splitting point derive from District 3 substation (Ngu Hanh Son) | 2 | x | 12 |  | 24 |  |  |  |
| 9 | Doc Soi – Quang Ngai | 2 | x | 60 |  | 120 |  |  | Hang circuit 1 |
| 10 | Tam Hiep – Doc Soi 500kv substation | 2 | x | 20 |  | 40 |  |  |  |
| 11 | Son Ha - Doc Soi 500kV substation | 2 | x | 50 |  | 100 |  |  |  |
| 12 | Quang Ngai – Quy Nhon | 2 | x | 143 |  | 286 |  |  |  |
| 13 | Establishing connections to Song Bung 2 HPS and Song Bung 4 HPS | 2 | x | 20 |  | 40 |  |  |  |
| 14 | Dak My 1 – Dak My 4 | 2 | x | 15 |  | 30 |  |  |  |
| 15 | Dak My 4 – Thanh My 500kV substation | 2 | x | 50 |  | 100 |  |  |  |
| 16 | Establishing connection between the group of HPSs in Se San and 220kV substation of Se San 4 HPS | 2 | x | 33 |  | 66 |  |  | Connected to Ha Se San 2 HPS, Se San 1 HPS |
| 17 | Buon Kuop – Dak Nong | 1 | x | 85 |  | 85 |  |  |  |
| 18 | PleiKu – Kon Tum | 2 | x | 30 |  | 60 |  |  | Under construction |
| 19 | Thuong Kon Tum – Quang Ngai | 2 | x | 76 |  | 152 |  |  |  |
| 20 | An Khe – An Nhon | 2 | x | 30 |  | 60 |  |  | For circuit 2 |
| 21 | Establishing 220kv connection to Van Phong TPP – Ninh Hoa switching substation | 2 | x | 20 |  | 40 |  |  | Using big cable |
| 22 | Splitting point Ninh Hoa switching substation - Ninh Hoa switching substation | 4 | x | 6 |  | 24 |  |  | Using 4-circuit towers |
| 23 | Splitting point An Nhon | 2 | x | 6 |  | 12 |  |  |  |
| 24 | Tuy Hoa – Nha Trang | 2 | x | 147 |  | 294 |  |  | For circuit 2 |
| 25 | Nha Trang - Cam Ranh | 2 | x | 60 |  | 120 |  |  |  |
| 26 | Krong Buk - Nha Trang | 1 | x | 147 |  | 147 |  |  | Circuit 2 started building in quarter III/2010 |
| 27 | Splitting point Van Phong | 2 | x | 2 |  | 4 |  |  |  |
| 28 | Pleiku - Krong Buk | 1 | x | 143 |  | 143 |  |  | Replace with big cable |
|  | Southern area |  |  |  |  | 3906 |  |  |  |
| 1 | Cam Ranh – Thap Cham | 2 | x | 45 |  | 90 |  |  |  |
| 2 | Thap Cham – Vinh Tan | 2 | x | 44 |  | 88 |  |  | Connected to Vinh Tan TPP |
| 3 | Thap Cham – Da Nhim | 1 | x | 40 |  | 40 |  |  |  |
| 4 | Vinh Tan – Phan Thiet | 2 | x | 100 |  | 200 |  |  | Connected to Vinh Tan TPP |
| 5 | Phan Thiet - Ham Tan | 2 | x | 63 |  | 126 |  |  | Connected to Vinh Tan TPP |
| 6 | Ham Tan – Chau Duc | 2 | x | 62 |  | 124 |  |  | Connected to Vinh Tan TPP |
| 7 | Chau Duc - Phu My 2 industrial zone | 2 | x | 21 |  | 42 |  |  | Connected to Vinh Tan TPP |
| 8 | Splitting point - Phu My 2 industrial zone | 2 | x | 14 |  | 28 |  |  |  |
| 9 | Ba Ria – Vung Tau | 2 | x | 14 |  | 28 |  |  |  |
| 10 | Binh Long - Tay Ninh 2 | 2 | x | 64 |  | 128 |  |  |  |
| 11 | Dong Nai 2 – Di Linh | 2 | x | 15 |  | 30 |  |  | Connected to Dong Nai 2 HPS |
| 12 | Di Linh – Da Nhim | 1 | x | 80 |  | 80 |  |  | For circuit 2 |
| 13 | Di Linh – Tan Rai | 2 | x | 10 |  | 20 |  |  | Connected to Aluminum processing |
| 14 | Song May – Uyen Hung | 2 | x | 21 |  | 42 |  |  |  |
| 15 | Uyen Hung - Tan Dinh | 2 | x | 18 |  | 36 |  |  |  |
| 16 | Song May - Bao Loc | 1 | x | 124 |  | 124 |  |  | For circuit 2 (Upgrading existing lines to double circuit. shall be considered) |
| 17 | Di Linh – Bao Loc | 1 | x | 37 |  | 37 |  |  | For circuit 2 (Upgrading existing lines to double circuit. shall be considered) |
| 18 | Ham Thuan – Bao Loc | 1 | x | 39 |  | 39 |  |  | For circuit 2 |
| 19 | Duc Trong – splitting point Da Nhim – Di Linh | 2 | x | 2 |  | 4 |  |  |  |
| 20 | Splitting point Xuan Loc | 4 | x | 5 |  | 20 |  |  | Using 4-circuit towers |
| 21 | Splitting point Hiep Binh Phuoc | 2 | x | 2 |  | 4 |  |  |  |
| 22 | Splitting point Nam Sai Gon | 4 | x | 3 |  | 12 |  |  | Using 4-circuit towers |
| 23 | Splitting point Binh Tan | 4 | x | 3 |  | 12 |  |  | Using 4-circuit towers |
| 24 | Splitting point Thuan An | 4 | x | 3 |  | 12 |  |  | Using 4-circuit towers |
| 25 | Song May – splitting point Tri An – Long Binh | 2 | x | 5 |  | 10 |  |  |  |
| 26 | Song May – splitting point Bao Loc – Long Binh | 4 | x | 10 |  | 40 |  |  | Using 4-circuit towers |
| 27 | Splitting point Song May – Long Binh | 2 | x | 15 |  | 30 |  |  | Upgrade 2 circuits |
| 28 | Splitting point – My Xuan | 4 | x | 2 |  | 8 |  |  | Using 4-circuit towers |
| 29 | Splitting point Phu My - Ba Ria - Phu My 2 industrial zone | 4 | x | 2 |  | 8 |  |  | Using 4-circuit towers |
| 30 | Thu Duc Bac – splitting point Thu Duc – Long Binh | 4 | x | 3 |  | 12 |  |  | Using 4-circuit towers |
| 31 | Tan Uyen 500kV substation - splitting point Thu Duc Bac – Thu Duc | 4 | x | 8 |  | 32 |  |  | Using 4-circuit towers |
| 32 | Tan Uyen 500kV substation - Uyen Hung | 2 | x | 15 |  | 30 |  |  |  |
| 33 | Dak Nong – Phuoc Long – Binh Long | 2 | x | 130 |  | 260 |  |  |  |
| 34 | Ben Cat – splitting point My Phuoc – Binh Long | 2 | x | 11 |  | 22 |  |  |  |
| 35 | Cu Chi – splitting point Tan Dinh – Trang Bang | 4 | x | 3 |  | 12 |  |  | Using 4-circuit towers |
| 36 | Cau Bong 500kV substation – Hoc Mon | 6 | x | 16 |  | 96 |  |  | Using 6-circuit towers, with 2 110kv circuits |
| 37 | Cau Bong 500kV substation – Cu Chi | 6 | x | 22 |  | 132 |  |  | Using 6-circuit towers, with 2 110kv circuits |
| 38 | Establishing connection between Vinh Loc to 2 circuits Cau Bong – Hoc Mon | 2 | x | 5 |  | 10 |  |  |  |
| 39 | Splitting point Vinh Loc – Phu Lam – Binh Tan | 4 | x | 3 |  | 12 |  |  | Using 4-circuit towers |
| 40 | Nha Be – Phu Lam | 2 | x | 15 |  | 30 |  |  | Replace with big cable |
| 41 | Phu Lam – Hoc Mon | 2 | x | 19 |  | 38 |  |  | Replace with big cable |
| 42 | Cat Lai – Thu Duc | 2 | x | 9 |  | 18 |  |  | Replace with big cable |
| 43 | Tan Cang – Cat Lai | 2 | x | 12 |  | 24 |  |  | Install underground 220kV line with 3 circuits |
| 44 | Nam Sai Gon – District 8 | 2 | x | 6 |  | 12 |  |  | Install underground 220kV line |
| 45 | Duc Hoa 500kV substation – Duc Hoa 1 | 4 | x | 22 |  | 88 |  |  | Using 4-circuit towers |
| 46 | Duc Hoa 1 - Cu Chi | 4 | x | 8 |  | 32 |  |  | Using 4-circuit towers |
| 47 | Duc Hoa 500kV substation – splitting point Phu Lam – Long An | 2 | x | 20 |  | 40 |  |  |  |
| 48 | Nhon Trach TPP - Nhon Trach city | 2 | x | 12 |  | 24 |  |  |  |
| 49 | Trang Bang - Tay Ninh | 2 | x | 44 |  | 88 |  |  |  |
| 50 | Tay Ninh – KamPong Cham | 2 | x | 116 |  | 232 |  |  | For Cambodia |
| 51 | Ben Luc – splitting point Phu Lam – Long An | 2 | x | 5 |  | 10 |  |  |  |
| 52 | Can Duoc - Splitting point Phu My – My Tho | 4 | x | 7 |  | 28 |  |  | Using 4-circuit towers |
| 53 | My Tho 500kV substation – splitting point – Cai Lay | 4 | x | 2 |  | 8 |  |  |  |
| 54 | My Tho 500kV substation – splitting point My Tho – Cai Lay | 4 | x | 2 |  | 8 |  |  |  |
| 55 | Duyen Hai TPP – Mo Cay | 2 | x | 77 |  | 154 |  |  | Connected to Duyen Hai TPP |
| 56 | Mo Cay – Ben Tre | 2 | x | 20 |  | 40 |  |  | Connected to Duyen Hai TPP |
| 57 | My Tho – Ben Tre | 1 | x | 18 |  | 18 |  |  | Under construction |
| 58 | Duyen Hai TPP – Tra Vinh | 2 | x | 45 |  | 90 |  |  | Connected to Duyen Hai TPP |
| 59 | Vinh Long – Tra Vinh | 2 | x | 62 |  | 124 |  |  | Connected to Duyen Hai TPP |
| 60 | Sa Dec Industrial Zone – splitting point Vinh Long 2 – O Mon | 2 | x | 5 |  | 10 |  |  |  |
| 61 | Cao Lanh – Cai Lay | 1 | x | 54 |  | 54 |  |  | Double-circuit line shall be considered |
| 62 | Cao LAnh – Thot Not | 1 | x | 27 |  | 27 |  |  | Double-circuit line shall be considered |
| 63 | Long Phu TPP – Soc Trang | 4 | x | 25 |  | 100 |  |  | Connected to Long Phu TPP |
| 64 | Long Phu TPP – Can Tho – Tra Noc | 2 | x | 95 |  | 190 |  |  | Connected to Long Phu TPP |
| 65 | Phung Hiep – splitting point O Mon – Soc Trang | 4 | x | 6 |  | 24 |  |  |  |
| 66 | Long Xuyen 2 – splitting point Chau Doc – Thot Not | 4 | x | 5 |  | 20 |  |  |  |
| 67 | Ca Mau TPP – Ca Mau | 1 | x | 5 |  | 5 |  |  | Circuit 2 |
| 68 | Phu Lam – Cai Lay | 2 | x | 70 |  | 140 |  |  | Replace with big cable |
| 69 | Phu My - Long Thanh | 2 | x | 25 |  | 50 |  |  | Replace with big cable |
| 70 | Long Thanh – Long Binh | 4 | x | 25 |  | 100 |  |  | Replace with big cable |
|  | Lines built over the period from 2016 to 2020Lines built over the period from 2016 to 2020Lines built over the period from 2016 to 2020Lines built over the period from 2016 to 2020Lines built over the period from 2016 to 2020Lines built over the period from 2016 to 2020 |  |  |  |  |  |  |  |  |
|  | Northern area |  |  |  |  | 1584 |  |  |  |
| 1 | Mai Dong – Tay Ho | 2 | x | 15 |  | 30 |  |  | Underground cable |
| 2 | Trung Son HPS - splitting pointed Hoa Binh – Nho Quan | 2 | x | 77 |  | 154 |  |  |  |
| 3 | Me Linh – splitting point Soc Son – Van Tri | 2 | x | 5 |  | 10 |  |  |  |
| 4 | Dong Anh - Dong Anh 2 | 2 | x | 20 |  | 40 |  |  |  |
| 5 | Connection - Dong Anh 500kV substation | 4 | x | 5 |  | 20 |  |  |  |
| 6 | Vinh Tuong – splitting point Viet Tri - Son Tay | 2 | x | 3 |  | 6 |  |  |  |
| 7 | Tay Ha Noi – Me Linh | 2 | x | 40 |  | 80 |  |  |  |
| 8 | Replacing with thermotolerant cable – Ha Dong - Chem | 2 | x | 20 |  | 40 |  |  | Connected to Tay Ha Noi 500kV Substation |
| 9 | Van Dien – splitting point Thuong Tin – Ha Dong | 2 | x | 6 |  | 12 |  |  |  |
| 10 | Chuong My – splitting point Hoa Binh – Ha Dong | 2 | x | 5 |  | 10 |  |  |  |
| 11 | Ung Hoa – splitting point Ha Dong – Phu Ly | 2 | x | 5 |  | 10 |  |  |  |
| 12 | Ung Hoa – Phu Ly - Nho Quan | 1 | x | 65 |  | 65 |  |  | For circuit 2 (Upgrading existing lines to double circuit. shall be considered) |
| 13 | Hai Ha – Cam Pha | 2 | x | 85 |  | 170 |  |  | For circuit 2 |
| 14 | An Lao - Hai Phong 500kV substation | 2 | x | 5 |  | 10 |  |  |  |
| 15 | Connection to Hai Phong 500kV substation | 4 | x | 2 |  | 8 |  |  |  |
| 16 | Tam Hung – Bac Song Gam | 2 | x | 12 |  | 24 |  |  |  |
| 17 | Gia Loc – splitting point Hai Duong TPP – Pho Noi | 2 | x | 5 |  | 10 |  |  |  |
| 18 | Lines Connected to Hai Duong 3 switching substation | 6 | x | 4 |  | 24 |  |  | 03 double-circuit lines |
| 19 | Pho Cao – splitting point Kim Dong - Thai Binh | 2 | x | 8 |  | 16 |  |  |  |
| 20 | Yen My - splitting point Thuong Tin – Pho Noi | 2 | x | 7 |  | 14 |  |  |  |
| 21 | Nho Quan – Thanh Nghi | 2 | x | 25 |  | 50 |  |  |  |
| 22 | Ly Nhan – Thanh Nghi | 2 | x | 23 |  | 46 |  |  |  |
| 23 | Thai Binh – Nam Dinh | 1 | x | 25 |  | 25 |  |  | For Circuit 2 |
| 24 | Nam Dinh - Ninh Binh | 1 | x | 31 |  | 31 |  |  | Double-circuit line shall be considered |
| 25 | Hua Na HPS - splitting point Thanh Hoa – Bim Son | 2 | x | 70 |  | 140 |  |  |  |
| 26 | Thai Binh – Thai Binh 500/220kV line | 2 | x | 15 |  | 30 |  |  |  |
| 27 | Vu Thu – splitting point Thai Binh – Truc Ninh | 2 | x | 5 |  | 10 |  |  |  |
| 28 | Splitting point – Quynh Phu | 2 | x | 5 |  | 10 |  |  |  |
| 29 | Ninh Binh – Thanh Hoa | 1 | x | 62 |  | 62 |  |  | For circuit 2 (Upgrading existing lines to double circuit. shall be considered) |
| 30 | Bac Me HPS – splitting point Ha Giang - Tuyen Quang HPS | 2 | x | 5 |  | 10 |  |  |  |
| 31 | Cao Bang – Lang Son | 1 | x | 100 |  | 100 |  |  |  |
| 32 | Phu Binh – splitting point Bac Giang – Thai Nguyen | 2 | x | 5 |  | 10 |  |  |  |
| 33 | Viet Tri 500kV substation – Ba Thien | 2 | x | 38 |  | 76 |  |  |  |
| 34 | Bac Giang – Pha Lai | 1 | x | 27 |  | 27 |  |  | For circuit 2 (Upgrading existing lines to double circuit. shall be considered) |
| 35 | Bac Ninh – Bac Ninh 500kV substation | 2 | x | 10 |  | 20 |  |  |  |
| 36 | Bac Ninh 500kV substation – splitting point Pho Noi – Tien Son | 4 | x | 5 |  | 20 |  |  |  |
| 37 | Thua– splitting point Pho Noi – Tien Son | 2 | x | 4 |  | 8 |  |  |  |
| 38 | Lach Huyen Steel – Yen Hung | 2 | x | 18 |  | 36 |  |  |  |
| 39 | Ngoc Lac – splitting point Hua Na – Thanh Hoa | 2 | x | 30 |  | 60 |  |  |  |
| 40 | Nong Cong – Thanh Hoa | 2 | x | 25 |  | 50 |  |  |  |
| 41 | Splitting point Nam Cam substation | 2 | x | 5 |  | 10 |  |  |  |
|  | Central area |  |  |  |  | 1546 |  |  |  |
| 1 | Dong Ha – Lao Bao | 2 | x | 52 |  | 104 |  |  |  |
| 2 | Connecting 220kV line to Thua Thien Hue 500kV substation | 4 | x | 5 |  | 20 |  |  |  |
| 3 | Ha Se San 2 - splitting point Se San 4 – Pleiku | 2 | x | 190 |  | 380 |  |  |  |
| 4 | Cau Hai– splitting point Hue – Hoa Khanh | 4 | x | 5 |  | 20 |  |  |  |
| 5 | Splitting point – Lien Chieu | 4 | x | 6 |  | 24 |  |  |  |
| 6 | District 3 - An Don | 2 | x | 6 |  | 12 |  |  | Underground cable shall be considered |
| 7 | District 3 – Hoi An | 2 | x | 15 |  | 30 |  |  |  |
| 8 | Splitting point Hoai Nhon – Hoai Nhon | 4 | x | 5 |  | 20 |  |  | Using 4-circuit towers |
| 9 | Quang Ngai - Doc Soi 500kV substation | 2 | x | 29 |  | 58 |  |  | For circuit 2 |
| 10 | Quang Ngai – An Nhon | 2 | x | 143 |  | 286 |  |  | For circuit 2 |
| 11 | Splitting point Song Cau – Song Cau | 6 | x | 5 |  | 30 |  |  |  |
| 12 | An Khe – An Nhon | 1 | x | 146 |  | 146 |  |  | For circuit 2 |
| 13 | An Nhon – Tuy Hoa | 2 | x | 98 |  | 196 |  |  |  |
| 14 | Splitting point Khanh Hoa – Khanh Hoa | 4 | x | 5 |  | 20 |  |  | Using 4-circuit towers |
| 15 | Establishing connections to HPSs in Southern Laos and Cambodia | 2 | x | 100 |  | 200 |  |  |  |
|  | Southern area |  |  |  |  | 2175 |  |  |  |
| 1 | Ninh Phuoc – splitting point Vinh Tan – Thap Cham | 4 | x | 2 |  | 8 |  |  |  |
| 2 | Phan Ri – splitting point Vinh Tan – Phan Thiet | 4 | x | 2 |  | 8 |  |  |  |
| 3 | Splitting point Tan Duc | 2 | x | 5 |  | 10 |  |  |  |
| 4 | Splitting point Dat Do | 4 | x | 2 |  | 8 |  |  |  |
| 5 | Phu My - Phu My 2 industrial zone | 2 | x | 20 |  | 40 |  |  | Phu My – Ba Ria 110 kV line shall be considered to be used |
| 6 | Splitting point Phu My 3 industrial zone | 4 | x | 3 |  | 12 |  |  |  |
| 7 | Dong Nai 500kV substation – splitting point Chau Duc - Phu My 2 industrial zone | 4 | x | 20 |  | 80 |  |  |  |
| 8 | Thu Thiem – splitting point Cat Lai – Tan Cang | 4 | x | 3 |  | 12 |  |  | 220kV underground cable |
| 9 | Phu Lam – Dam Sen | 3 | x | 6 |  | 18 |  |  | 220kV underground cable |
| 10 | Tan Son Nhat - Hiep Binh Phuoc | 2 | x | 8 |  | 16 |  |  | 220kV underground cable |
| 11 | Hiep Binh Phuoc – Tan Uyen 500kV Substation | 2 | x | 14 |  | 28 |  |  | Using 4-circuit towers. 2 circuits shall be hung first |
| 12 | District 9 – splitting point Tan Uyen – Thu Duc | 4 | x | 2 |  | 8 |  |  | Using 4-circuit towers |
| 13 | South Hiep Phuoc - splitting point Thu Duc – Hoc Mon | 4 | x | 2 |  | 8 |  |  | Using 4-circuit towers |
| 14 | Long Thanh 500kv substation – splitting point Cat Lai – Thu Duc | 4 | x | 17 |  | 68 |  |  | Using 4-circuit towers |
| 15 | Nhon Trach TPP - Nhon Trach city | 2 | x | 12 |  | 24 |  |  |  |
| 16 | Long Thanh Industrial Zone - Dong Nai 500kV substation – Long Thanh | 4 | x | 2 |  | 8 |  |  | Using 4-circuit towers |
| 17 | Tam Phuoc – Long Thanh 500kV substation – Long Binh | 4 | x | 2 |  | 8 |  |  |  |
| 18 | An – brance Long Thanh 500kV substation – Long Thanh | 4 | x | 2 |  | 8 |  |  |  |
| 19 | Tan Dinh 500kV substation – splitting point Binh Hoa – Thuan An | 2 | x | 11 |  | 22 |  |  |  |
| 20 | Splitting point Tay Bac Cu Chi | 2 | x | 3 |  | 6 |  |  |  |
| 21 | Splitting point Phu Hoa Dong | 2 | x | 3 |  | 6 |  |  |  |
| 22 | Cau Bong 500kV substation – Binh Chanh 1 | 2 | x | 13 |  | 26 |  |  |  |
| 23 | Binh Duong 1 500kV substation – splitting point Tan Dinh - Uyen Hung | 4 | x | 2 |  | 8 |  |  |  |
| 24 | Tan Uyen – splitting poit Binh Duong 1 500kV substation – My Phuoc | 4 | x | 3 |  | 12 |  |  |  |
| 25 | My Phuoc 500kV substation – splitting point My Phuoc – Binh Long | 4 | x | 2 |  | 8 |  |  |  |
| 26 | My Phuoc 500kV substation – Ben Cat | 2 | x | 3 |  | 6 |  |  |  |
| 27 | Lai Uyen – splitting point My Phuoc – Binh Long | 4 | x | 3 |  | 12 |  |  |  |
| 28 | Phuoc Long – splitting point Binh Long - Dak Nong | 4 | x | 2 |  | 8 |  |  |  |
| 29 | Tay Ninh 2 – splitting point Trang Bang – Tay Ninh | 4 | x | 2 |  | 8 |  |  |  |
| 30 | Sam Bo HPS – Tan Bien | 2 | x | 170 |  | 340 |  |  |  |
| 31 | Tan Bien – splitting point Tay Ninh 2 – KamPongCham | 4 | x | 150 |  | 600 |  |  |  |
| 32 | Sa Dec Industrial Zone – O Mon | 2 | x | 28 |  | 56 |  |  | Replace with big cable |
| 33 | Tan An – splitting point Can Duoc– My Tho | 4 | x | 5 |  | 20 |  |  |  |
| 34 | Go Cong – Can Duoc | 2 | x | 22 |  | 44 |  |  |  |
| 35 | Vinh Long 3 – splitting point Tra Vinh – Vinh Long 2 | 4 | x | 3 |  | 12 |  |  |  |
| 36 | Lap Vo – Thot Not | 2 | x | 12 |  | 24 |  |  |  |
| 37 | Chau Thanh – splitting point Long Xuyen 2 – Chau Doc | 4 | x | 2 |  | 8 |  |  |  |
| 38 | My Tu – splitting point Phung Hiep – Soc Trang | 2 | x | 12 |  | 24 |  |  |  |
| 39 | Gia Rai – splitting point Bac Lieu 2 – Ca Mau | 4 | x | 2 |  | 8 |  |  |  |
| 40 | Ngoc Hien – Ca Mau | 2 | x | 55 |  | 110 |  |  |  |
| 41 | Cai Nuoc – Ca Mau | 2 | x | 45 |  | 90 |  |  |  |
| 42 | Go Quao – splitting point Ca Mau – Rach Gia | 2 | x | 6 |  | 12 |  |  |  |
| 43 | Vi Thanh – splitting point Ca Mau – Bac Lieu 2 | 2 | x | 8 |  | 16 |  |  |  |
| 44 | Kien Luong TPP – Kien Binh 2 | 2 | x | 10 |  | 20 |  |  | Connected to Kien Luong TPP |
| 45 | Kien Luong TPP – Chau Doc | 3 | x | 99 |  | 297 |  |  | Connect to Kien Luong TPP (using 4-circuit tower at the beginning part) |

APPENDIX V

LIST OF ELECTRICAL GRID PROJECTS EXPECTED TO BE COMMENCED OVER THE PERIOD FROM 2021 TO 2030 
(Enclosed with the Decision No. 1208/QĐ-TTg dated July 21, 2011 of the Prime Minister)

500kV SUBSTATIONS BUILT OVER THE PERIOD FROM 2021 TO 2030

| No | Project name | Number of transformers x MVA |  |  |  | Number of transformers x MVA Number of transformers x MVA Capacity - MVA |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Substations built over the period from 2021 to 2025Substations built over the period from 2021 to 2025Substations built over the period from 2021 to 2025Substations built over the period from 2021 to 2025Substations built over the period from 2021 to 2025Substations built over the period from 2021 to 2025 |  |  |  |  |  |  |  |
|  | Northern area |  |  |  |  | Northern areaNorthern areaNorthern area12700 |  |  |
| 1 | Quy Nhon | 1 | x | 900 | 900 |  | Replace transformer 2 |  |
| 2 | Pho Noi | 1 | x | 900 | 900 |  | Replace transformer 1 |  |
| 3 | Viet Tri | 1 | x | 900 | 900 |  | Install transformer 2 |  |
| 4 | Vinh Yen converter station |  |  |  | 1000 |  | For 1000 MW |  |
| 5 | Tay Ha Noi | 1 | x | 900 | 900 |  | Install transformer 2 |  |
| 6 | Thai Binh | 1 | x | 600 | 600 |  | Install a transformer in the switching substation |  |
| 7 | Hai Phong | 1 | x | 900 | 900 |  | Install transformer 2 |  |
| 8 | Thai Nguyen | 2 | x | 900 | 1800 |  | Newly constructed |  |
| 9 | Vinh Yen | 2 | x | 900 | 1800 |  | Newly constructed |  |
| 10 | Bac Giang | 2 | x | 600 | 1200 |  | Newly constructed (with 03 transformers) |  |
| 11 | Thanh Hoa | 2 | x | 900 | 1800 |  | Newly constructed |  |
|  | Central area |  |  |  |  | Central areaCentral areaCentral area900 |  |  |
| 1 | Da Nang | 1 | x | 900 | 900 |  | Replace transformer 2 |  |
|  | Southern area |  |  |  |  | Southern areaSouthern areaSouthern area10800 |  |  |
| 1 | DI Linh | 1 | x | 450 | 450 |  | Install transformer 2 |  |
| 2 | Tan Uyen | 1 | x | 900 | 900 |  | Install transformer 3 |  |
| 3 | Dong Nai | 2 | x | 900 | 1800 |  | Install a transformer in the switching substation |  |
| 4 | Tan Dinh | 1 | x | 900 | 900 |  | Replace transformer 1 |  |
| 5 | My Phuoc | 2 | x | 900 | 1800 |  | Install transformer 2, 3 |  |
| 6 | Binh Duong 1 | 1 | x | 900 | 900 |  | Install transformer 2 |  |
| 7 | Song May | 2 | x | 900 | 1800 |  | Replace 2 125MVA transformers |  |
| 8 | Long Thanh | 1 | x | 900 | 900 |  | Install transformer 3 |  |
| 9 | Son My TPP | 1 | x | 450 | 450 |  | Powerline communication in the plant |  |
| 10 | Thot Not | 1 | x | 900 | 900 |  | Install transformer 2 |  |
| Substations built over the period from 2026 to 2030Substations built over the period from 2026 to 2030Substations built over the period from 2026 to 2030Substations built over the period from 2026 to 2030Substations built over the period from 2026 to 2030Substations built over the period from 2026 to 2030Substations built over the period from 2026 to 2030 |  |  |  |  |  |  |  |  |
|  | Northern area |  |  |  |  | Northern areaNorthern areaNorthern area7500 |  |  |
| 1 | Thai Binh | 1 | x | 900 | 900 |  | Install transformer 2 |  |
| 2 | Bac Giang | 1 | x | 600 | 600 |  | Install transformer 3 |  |
| 3 | Bac Ninh | 1 | x | 600 | 600 |  | Install transformer 3 |  |
| 4 | Ha Tinh | 2 | x | 900 | 1800 |  | Replace 2 transformers |  |
| 5 | Son Tay | 2 | x | 900 | 1800 |  | Newly constructed |  |
| 6 | South Ha Noi | 2 | x | 900 | 1800 |  | Newly constructed |  |
|  | Central area |  |  |  |  | Central areaCentral areaCentral area2.550 |  |  |
| 1 | Thua Thien Hue | 1 | x | 600 | 600 |  | Install transformer 2 |  |
| 2 | Doc Soi | 1 | x | 900 | 900 |  | Replace transformer 1 |  |
| 3 | Van Phong | 1 | x | 450 | 450 |  | Install transformer 2 |  |
| 4 | Binh Dinh TPP | 1 | x | 600 | 600 |  | Install transformer 1 |  |
|  | Southern area |  |  |  |  | Southern areaSouthern areaSouthern area10350 |  |  |
| 1 | Duyen Hai | 1 | x | 450 | 450 |  | Install transformer 2 |  |
| 2 | Long Phu | 1 | x | 450 | 450 |  | Install transformer 2 |  |
| 3 | Vinh Tan TPP | 1 | x | 450 | 450 |  | Install transformer 2 |  |
| 4 | Binh Duong 1 | 1 | x | 900 | 900 |  | Install transformer 3 |  |
| 5 | Cau Bong | 1 | x | 900 | 900 |  | Install transformer 3 |  |
| 6 | Cu Chi | 1 | x | 900 | 900 |  | Install transformer 3 |  |
| 7 | Tay Ninh | 2 | x | 900 | 1800 |  |  |  |
| 8 | Tien Giang | 2 | x | 900 | 1800 |  |  |  |
| 9 | Dong Thap 1 | 2 | x | 900 | 1800 |  |  |  |
| 10 | Thot Not | 1 | x | 900 | 900 |  | Replace transformer 1 |  |

500kV SUBSTATIONS BUILT OVER THE PERIOD FROM 2021 TO 2030

| No. | Project name | Number of Circuits x km |  |  |  | Number of Circuits x kmNumber of Circuits x kmkm |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Lines built over the period from 2021 to 2025Lines built over the period from 2021 to 2025Lines built over the period from 2021 to 2025Lines built over the period from 2021 to 2025Lines built over the period from 2021 to 2025Lines built over the period from 2021 to 2025 |  |  |  |  |  |  |  |
|  | Northern area |  |  |  | 680 |  |  |  |
| 1 | Hai Phong - Hai Phong 3 TPP | 2 | x | 45 | 90 |  | Connected to Hai Phong 3 TPP |  |
| 2 | Quynh Lap TPP – Vung Ang 3 TPP | 2 | x | 220 | 440 |  |  |  |
| 3 | Vung Ang 3 TPP – Quang Trach – Vung Ang 2 | 2 | x | 20 | 40 |  | For connecting to Vung Ang 3 TPP |  |
| 4 | Splitting point - Thai Binh | 2 | x | 5 | 10 |  | For connecting to new substation |  |
| 5 | Splitting point Viet Tri – Hiep Hoa - Vinh Yen | 2 | x | 5 | 10 |  | For connecting to new substation |  |
| 6 | Splitting point Hiep Hoa – Quang Ninh – Bac Giang | 2 | x | 5 | 10 |  | For connecting to new substation |  |
| 7 | Hiep Hoa - Thai Nguyen | 2 | x | 40 | 80 |  |  |  |
|  | Southern area |  |  |  | 1554 |  |  |  |
| 1 | Ninh Thuan 2 NPP – My Phuoc | 2 | x | 292 | 584 |  | Connected to generator 1 of Ninh Thuan 2 NPP |  |
| 2 | Ninh Thuan 1 NPP – Binh Duong 1 | 2 | x | 250 | 500 |  | Connected to the generator 3, 4 of Ninh Thuan 1 NPP |  |
| 3 | Kien Luong TPP – Cu Chi | 2 | x | 235 | 470 |  | Connected to Kien Luong TPP |  |
|  | Lines built over the period from 2026 to 2030Lines built over the period from 2026 to 2030Lines built over the period from 2026 to 2030Lines built over the period from 2026 to 2030Lines built over the period from 2026 to 2030Lines built over the period from 2026 to 2030 |  |  |  |  |  |  |  |
|  | Northern area |  |  |  | 760 |  |  |  |
| 1 | Splitting point – Son Tay | 2 | x | 25 | 50 |  |  |  |
| 2 | Establishing connection to Northern CPP | 2 | x | 300 | 600 |  |  |  |
| 3 | Southern Ha Noi – splitting point Nho Quan – Thuong Tin | 2 | x | 5 | 10 |  | For connecting to new substation |  |
| 4 | Splitting point – Son Tay | 4 | x | 5 | 20 |  | For connecting to new substation |  |
| 5 | Vinh Yen – Son Tay | 2 | x | 40 | 80 |  |  |  |
|  | Central area |  |  |  | 920 |  |  |  |
| 1 | Van Phong TPP – Binh Dinh TPP | 1 | x | 220 | 220 |  | Double circuit line shall be considered |  |
| 2 | Doc Soi – Binh Dinh TPP | 2 | x | 200 | 400 |  |  |  |
| 3 | Establishing connection to Central NPPs | 2 | x | 150 | 300 |  |  |  |
|  | Southern area |  |  |  | 1044 |  |  |  |
| 1 | Ninh Thuan 2 NPP – My Phuoc | 2 | x | 292 | 584 |  | Connected to the generator 3, 4 of Ninh Thuan 2 NPP |  |
| 2 | Cu Chi – Tay Ninh | 2 | x | 50 | 100 |  |  |  |
| 3 | Establishing connection to Southern CCP | 2 | x | 100 | 200 |  |  |  |
| 4 | Duc Hoa – My Tho | 2 | x | 50 | 100 |  | Connect to O Mon – My Tho line (not to My Tho substation) |  |
| 5 | Establishing connection to Tien Giang | 8 | x | 5 | 40 |  |  |  |
| 6 | Establishing connection to Dong Thap 1 | 4 | x | 5 | 20 |  | Transitted from 2 circuits of current 500kv lines |  |

220kV SUBSUBSTATIONS BUILT OVER THE PERIOD FROM 2021 TO 2030

| No | Project name | Number of transformers x MVA |  |  | Number of transformers x MVA Number of transformers x MVA Capacity - MVA | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | Subsubstations built over the period from 2021 to 2025Subsubstations built over the period from 2021 to 2025Subsubstations built over the period from 2021 to 2025Subsubstations built over the period from 2021 to 2025Subsubstations built over the period from 2021 to 2025Subsubstations built over the period from 2021 to 2025 |  |  |  |  |  |
|  | Northern area |  |  |  | Northern areaNorthern areaNorthern area17400 |  |
| 1 | An Duong (Tay Ho) | 1 | x | 250 | 250 | Install transformer 3 |
| 2 | Soc Son 2 | 2 | x | 250 | 500 | Newly constructed |
| 3 | Chuong My | 1 | x | 250 | 250 | Install transformer 2 |
| 4 | Hoa Lac | 2 | x | 250 | 500 | Newly constructed |
| 5 | Me Linh | 1 | x | 250 | 250 | Install transformer 2 |
| 6 | Hai Duong 1 | 1 | x | 250 | 250 | Replace transformer 1 |
| 7 | Gia Loc | 1 | x | 250 | 250 | Install transformer 2 |
| 8 | Pho Cao | 1 | x | 250 | 250 | Install transformer 2 |
| 9 | Yen My | 1 | x | 250 | 250 | Install transformer 2 |
| 10 | Thanh Nghi | 1 | x | 250 | 250 | Install transformer 2 |
| 11 | Ly Nhan | 1 | x | 250 | 250 | Install transformer 2 |
| 12 | Nam Dinh 2 | 2 | x | 125 | 250 | Newly constructed |
| 13 | Vu Thu | 1 | x | 250 | 250 | Install transformer 2 |
| 14 | Quynh Phu | 1 | x | 250 | 250 | Install transformer 2 |
| 15 | Ninh Binh 2 | 2 | x | 250 | 500 | Replace transformer |
| 16 | Cao Bang | 1 | x | 250 | 250 | Replace transformer 1 |
| 17 | Lang Son | 1 | x | 250 | 250 | Install transformer 2 |
| 18 | Lao Cai | 1 | x | 250 | 250 | Replace transformer 1 |
| 19 | Bao Thang | 1 | x | 250 | 250 | Install transformer 2 |
| 20 | Tuyen Quang | 1 | x | 250 | 250 | Replace transformer 1 |
| 21 | Yen Bai | 1 | x | 250 | 250 | Replace transformer 2 |
| 22 | Nghia Lo | 1 | x | 125 | 125 | Install transformer 2 |
| 23 | Phu Binh | 1 | x | 250 | 250 | Install transformer 2 |
| 24 | Phu Tho | 1 | x | 250 | 250 | Install transformer 2 |
| 25 | Vinh Tuong | 1 | x | 250 | 250 | Install transformer 2 |
| 26 | Tan Yen | 1 | x | 250 | 250 | Install transformer 2 |
| 27 | Hoanh Bo | 1 | x | 250 | 250 | Replace transformer 2 |
| 28 | Hai Ha industrial zone | 1 | x | 250 | 250 | Install transformer 2 |
| 29 | Hai Ha Steel | 2 | x | 200 | 400 | Newly constructed |
| 30 | Lach Huyen Steel | 2 | x | 125 | 250 | Install transformer 2 |
| 31 | Lai Chau | 1 | x | 125 | 125 | Install transformer 2 |
| 32 | Son La commune | 1 | x | 250 | 250 | Replace transformer 2 |
| 33 | Hoa Binh | 1 | x | 250 | 250 | Replace transformer |
| 34 | Sam Son | 1 | x | 250 | 250 | Install transformer 2 |
| 35 | Hau Loc | 1 | x | 125 | 125 | Install transformer 2 |
| 36 | Do Luong | 2 | x | 250 | 500 | Replace transformer |
| 37 | Cua Lo | 1 | x | 125 | 125 | Install transformer 2 |
| 38 | Thach Khe | 1 | x | 250 | 250 | Replace transformer |
| 39 | Long Bien 2 | 1 | x | 250 | 250 | Newly constructed |
| 40 | Dong Anh 3 | 1 | x | 250 | 250 | Newly constructed |
| 41 | Soc Son 3 | 1 | x | 250 | 250 | Newly constructed |
| 42 | Phuc Tho | 1 | x | 250 | 250 | Newly constructed |
| 43 | Dai Ban | 2 | x | 250 | 500 | Newly constructed |
| 44 | Vinh Bao | 2 | x | 250 | 500 | Newly constructed |
| 45 | Do Son | 2 | x | 250 | 500 | Newly constructed |
| 46 | Thanh Ha | 1 | x | 250 | 250 | Newly constructed |
| 47 | Hung Yen City | 1 | x | 250 | 250 | Newly constructed |
| 48 | Hai Hau | 1 | x | 250 | 250 | Newly constructed |
| 49 | An Hoa | 1 | x | 250 | 250 | Newly constructed |
| 50 | Tam Diep | 1 | x | 250 | 250 | Newly constructed |
| 51 | Bac Quang | 1 | x | 125 | 125 | Newly constructed |
| 52 | Dong Mo | 1 | x | 250 | 250 | Newly constructed |
| 53 | Ham Yen | 1 | x | 125 | 125 | Newly constructed |
| 54 | Dai Tu | 1 | x | 250 | 250 | Newly constructed |
| 55 | Pho Vang | 1 | x | 250 | 250 | Newly constructed |
| 56 | Tam Duong | 1 | x | 250 | 250 | Newly constructed |
| 57 | Chu | 1 | x | 250 | 250 | Newly constructed |
| 58 | Bac Giang 500kV | 1 | x | 250 | 250 | Newly constructed |
| 59 | Vo Cuong | 2 | x | 250 | 500 | Newly constructed |
| 60 | Thuan Thanh | 2 | x | 250 | 500 | Newly constructed |
| 61 | Nong Cong | 1 | x | 250 | 250 | Newly constructed |
| 62 | Tuong Duong | 1 | x | 125 | 125 | Newly constructed |
| 63 | Hoang Mai | 1 | x | 125 | 125 | Newly constructed |
| 64 | Can Loc | 1 | x | 250 | 250 | Newly constructed |
|  | Central area |  |  |  | Central areaCentral areaCentral area6000 |  |
| 1 | Hon La | 1 | x | 250 | 250 | Install transformer 1 |
| 2 | Dong Hoi | 2 | x | 250 | 500 | Replace transformer |
| 3 | Cua Tung | 1 | x | 125 | 125 | Install transformer 1 |
| 4 | Chan May | 1 | x | 250 | 250 | Install transformer 2 |
| 5 | Phong Dien | 1 | x | 250 | 250 | Replace transformer |
| 6 | Hoa Khanh | 1 | x | 250 | 250 | Replace transformer |
| 7 | Da Nang 2 (Hai Chau) | 1 | x | 250 | 250 | Install transformer 1 |
| 8 | Tam Ky | 1 | x | 250 | 250 | Replace transformer |
| 9 | Bac Chu Lai | 1 | x | 125 | 125 | Install transformer 1 |
| 10 | Quang Ngai 2 | 1 | x | 125 | 125 | Install transformer 1 |
| 11 | Quang Ngai | 1 | x | 250 | 250 | Replace transformer |
| 12 | Doc Soi | 1 | x | 250 | 250 | Replace transformer |
| 13 | Dung Quat | 1 | x | 250 | 250 | Replace transformer |
| 14 | Quy Nhon | 1 | x | 250 | 250 | Replace transformer |
| 15 | Binh Dinh | 1 | x | 250 | 250 | Install transformer 1 |
| 16 | An Nhon | 1 | x | 250 | 250 | Install transformer 2 |
| 17 | Tuy Hoa | 1 | x | 250 | 250 | Replace transformer 2 |
| 18 | Song Cau | 1 | x | 125 | 125 | Install transformer 2 |
| 19 | Nha Trang | 1 | x | 250 | 250 | Replace transformer 2 |
| 20 | Ninh Hoa | 1 | x | 250 | 250 | Install transformer 1 |
| 21 | Kon Tum | 1 | x | 125 | 125 | Install transformer 2 |
| 22 | PleiKu | 1 | x | 250 | 250 | Replace transformer 1 |
| 23 | Chu Se | 1 | x | 125 | 125 | Install transformer 2 |
| 24 | KrongBuk | 1 | x | 250 | 250 | Replace transformer 1 |
| 25 | Dak Nong | 1 | x | 250 | 250 | Install transformer 2 |
| 26 | Buon Kuop | 1 | x | 250 | 250 | Replace transformer 1 |
|  | Southern area |  |  |  | Southern areaSouthern areaSouthern area19375 |  |
| 1 | Da Lat | 1 | x | 250 | 250 |  |
| 2 | Ninh Phuoc | 1 | x | 250 | 250 |  |
| 3 | Phan Thiet | 1 | x | 250 | 250 | Replace 01 125MVA transformer |
| 4 | Phan Ri | 1 | x | 250 | 250 |  |
| 5 | Tan Duc | 2 | x | 250 | 500 |  |
| 6 | Nha Be | 1 | x | 250 | 250 | Install transformer 3 |
| 7 | Tao Dan | 1 | x | 250 | 250 | Install transformer 3 |
| 8 | Vinh Loc | 1 | x | 250 | 250 | Install transformer 2 |
| 9 | Nam Sai Gon | 1 | x | 250 | 250 | Install transformer 3 |
| 10 | Hiep Binh Phuoc | 1 | x | 250 | 250 | Install transformer 3 |
| 11 | Binh Tan | 1 | x | 250 | 250 | Install transformer 3 |
| 12 | Cau Bong | 1 | x | 250 | 250 | Install transformer 3 |
| 13 | Binh Chanh 2 | 2 | x | 250 | 500 |  |
| 14 | Eastern District 9 | 2 | x | 250 | 500 |  |
| 15 | Tan Thoi Hiep | 2 | x | 250 | 500 |  |
| 16 | Tan Cang | 1 | x | 250 | 250 | Install transformer 3 |
| 17 | District 9 | 1 | x | 250 | 250 | Install transformer 2 |
| 18 | Phuoc Long | 1 | x | 250 | 250 | Install transformer 2 |
| 19 | Binh Long | 1 | x | 250 | 250 | Replace transformer 2 |
| 20 | Trang Bang | 1 | x | 250 | 250 | Install transformer 3 |
| 21 | Tan Bien | 2 | x | 250 | 500 |  |
| 22 | My Phuoc | 1 | x | 250 | 250 | Install transformer 3 |
| 23 | Uyen Hung | 1 | x | 250 | 250 | Install transformer 3 |
| 24 | Ben Cat | 1 | x | 250 | 250 | Install transformer 3 |
| 25 | Tan Dinh 2 | 1 | x | 250 | 250 | Install transformer 3 |
| 26 | Tam Binh | 2 | x | 250 | 500 |  |
| 27 | Son Tay | 1 | x | 250 | 250 |  |
| 28 | My Phuoc | 2 | x | 250 | 500 |  |
| 29 | Xuan Loc | 1 | x | 250 | 250 | Install transformer 3 |
| 30 | Thong Nhat | 2 | x | 250 | 500 |  |
| 31 | Dinh Quan | 2 | x | 250 | 500 |  |
| 32 | Nhon Trach Industrial Zone | 2 | x | 250 | 500 |  |
| 33 | Phu My 2 industrial zone | 1 | x | 250 | 250 | Install transformer 3 |
| 34 | Dat Do | 2 | x | 250 | 500 |  |
| 35 | Phu My 3 industrial zone | 1 | x | 250 | 250 | Install transformer 2 |
| 36 | Chau Duc 2 | 1 | x | 250 | 250 |  |
| 37 | Phu My City | 1 | x | 250 | 250 |  |
| 38 | Tan An | 1 | x | 250 | 250 | Install transformer 2 |
| 39 | Duc Hoa 2 | 2 | x | 250 | 500 |  |
| 40 | Duc Hoa 3 | 1 | x | 250 | 250 |  |
| 41 | Can Duoc | 1 | x | 250 | 250 | Install transformer 2 |
| 42 | Thanh Binh | 2 | x | 250 | 500 |  |
| 43 | Hong Ngu | 1 | x | 250 | 250 |  |
| 44 | Chau Thanh | 1 | x | 250 | 250 | Install transformer 2 |
| 45 | Cho Moi | 1 | x | 250 | 250 |  |
| 46 | Cai Be | 2 | x | 250 | 500 |  |
| 47 | Cai Be 2 | 1 | x | 250 | 250 |  |
| 48 | Vinh Long 3 | 1 | x | 250 | 250 | Install transformer 2 |
| 49 | Mo Cay | 1 | x | 250 | 250 | Install transformer 2 |
| 50 | Ba Tri | 1 | x | 250 | 250 |  |
| 51 | Go Quao | 1 | x | 125 | 125 | Install transformer 2 |
| 52 | Hon Dat | 1 | x | 250 | 250 |  |
| 53 | Tra Noc | 2 | x | 250 | 500 | Replace 02 transformers |
| 54 | O Mon | 1 | x | 250 | 250 | Replace 01 125MVA transformer |
| 55 | Ninh Kieu | 1 | x | 250 | 250 | Install transformer 2 |
| 56 | O Mon 2 | 1 | x | 125 | 125 |  |
| 57 | Vi Thanh | 1 | x | 125 | 125 | Install transformer 2 |
| 58 | Phung Hiep | 1 | x | 250 | 250 | Replace transformer 1 |
| 59 | Tra Vinh | 1 | x | 250 | 250 | Replace transformer 1 |
| 60 | Duyen Hai | 1 | x | 250 | 250 | Install transformer 2 |
| 61 | Soc Trang | 1 | x | 250 | 250 | Replace transformer 1 |
| 62 | My Tu | 1 | x | 125 | 125 | Install transformer 2 |
| 63 | Long Phu TPP | 1 | x | 125 | 125 | Install transformer 2 |
| 64 | Gia Rai | 1 | x | 125 | 125 | Install transformer 2 |
| 65 | Hong Dan | 1 | x | 125 | 125 |  |
| 66 | Tran Van Thoi | 1 | x | 250 | 250 |  |
|  | Subsubstations built over the period from 2026 to 2030Subsubstations built over the period from 2026 to 2030Subsubstations built over the period from 2026 to 2030Subsubstations built over the period from 2026 to 2030Subsubstations built over the period from 2026 to 2030Subsubstations built over the period from 2026 to 2030 |  |  |  |  |  |
|  | Northern area |  |  |  | Northern areaNorthern areaNorthern area21250 |  |
| 1 | Soc Son | 1 | x | 250 | 250 | Install transformer 3 |
| 2 | Van Tri | 1 | x | 250 | 250 | Install transformer 3 |
| 3 | Long Bien 2 | 1 | x | 250 | 250 | Install transformer 2 |
| 4 | Dong Anh 3 | 1 | x | 250 | 250 | Install transformer 2 |
| 5 | Soc Son 3 | 1 | x | 250 | 250 | Install transformer 2 |
| 6 | Phuc Tho | 1 | x | 250 | 250 | Install transformer 2 |
| 7 | Dai Ban | 1 | x | 250 | 250 | Install transformer 2 |
| 8 | Vinh Bao | 1 | x | 250 | 250 | Install transformer 2 |
| 9 | Do Son | 1 | x | 250 | 250 | Install transformer 2 |
| 10 | Thanh Ha | 1 | x | 250 | 250 | Install transformer 2 |
| 11 | Hung Yen City | 1 | x | 250 | 250 | Install transformer 2 |
| 12 | Ly Nhan | 1 | x | 250 | 250 | Replace transformer 1 |
| 13 | Nam Dinh 2 | 2 | x | 250 | 500 | Replace 2 transformers |
| 14 | Hai Hau | 1 | x | 250 | 250 | Install transformer 2 |
| 15 | An Hoa | 1 | x | 250 | 250 | Install transformer 2 |
| 16 | Nho Quan | 1 | x | 250 | 250 | Replace transformer 1 |
| 17 | Tam Diep | 1 | x | 250 | 250 | Install transformer 2 |
| 18 | Lang Son | 1 | x | 250 | 250 | Replace transformer T1 |
| 19 | Bac Quang | 1 | x | 125 | 125 | Install transformer 2 |
| 20 | Cao Bang | 1 | x | 250 | 250 | Replace transformer 2 |
| 21 | Lao Cai | 1 | x | 250 | 250 | Replace transformer 2 |
| 22 | Than Uyen | 2 | x | 250 | 500 | Replace transformer |
| 23 | Bac Kan | 1 | x | 250 | 250 | Install transformer 2 |
| 24 | Dong Mo | 1 | x | 250 | 250 | Install transformer 2 |
| 25 | Tuyen Quang | 1 | x | 250 | 250 | Replace transformer 2 |
| 26 | Ham Yen | 1 | x | 125 | 125 | Install transformer 2 |
| 27 | Nghia Lo | 1 | x | 250 | 250 | Replace transformer 1 |
| 28 | Dai Tu | 1 | x | 250 | 250 | Install transformer 2 |
| 29 | Pho Vang | 1 | x | 250 | 250 | Install transformer 2 |
| 30 | Tam Duong | 1 | x | 250 | 250 | Install transformer 2 |
| 31 | Chu | 1 | x | 250 | 250 | Install transformer 2 |
| 32 | Bac Giang 500kV | 1 | x | 250 | 250 | Install transformer 2 |
| 33 | Yen Phong (Bac Ninh -3) | 1 | x | 250 | 250 | Install transformer 3 |
| 34 | Vo Cuong | 1 | x | 250 | 250 | Install transformer 2 |
| 35 | Thuan Thanh | 1 | x | 250 | 250 | Install transformer 2 |
| 36 | Cam Pha | 2 | x | 250 | 500 | Replace 02 transformers |
| 37 | Lai Chau | 1 | x | 250 | 250 | Replace transformer 1 |
| 38 | Hoa Binh | 1 | x | 250 | 250 | Replace transformer 2 |
| 39 | Hau Loc | 2 | x | 250 | 500 | Replace transformer |
| 40 | Nong Cong | 1 | x | 250 | 250 | Install transformer 2 |
| 41 | Cua Lo | 2 | x | 250 | 500 | Replace transformer |
| 42 | Tuong Duong | 2 | x | 250 | 500 | Replace transformer 1 + install transformer 2 |
| 43 | Hoang Mai | 1 | x | 250 | 250 | Install transformer 2 |
| 44 | Ha Tinh | 1 | x | 250 | 250 | Replace transformer 2 |
| 45 | Thach Khe | 1 | x | 250 | 250 | Replace transformer 2 |
| 46 | Can Loc | 1 | x | 250 | 250 | Install transformer 2 |
| 47 | Phu Xuyen | 1 | x | 250 | 250 | Newly constructed |
| 48 | Xuan Mai Park State | 1 | x | 250 | 250 | Newly constructed |
| 49 | Hoa Lac 2 | 1 | x | 250 | 250 | Newly constructed |
| 50 | Long Bien 3 | 1 | x | 250 | 250 | Newly constructed |
| 51 | Tien Lang | 2 | x | 250 | 500 | Newly constructed |
| 52 | Hai Phong 3 TPP | 1 | x | 250 | 250 | Newly constructed |
| 53 | Tu Ky | 1 | x | 250 | 250 | Newly constructed |
| 54 | Thanh Mien | 1 | x | 250 | 250 | Newly constructed |
| 55 | Khoai Chau | 1 | x | 250 | 250 | Newly constructed |
| 56 | Van Giang | 2 | x | 250 | 500 | Newly constructed |
| 57 | Dong Van | 1 | x | 250 | 250 | Newly constructed |
| 58 | Tien Hai | 2 | x | 250 | 500 | Newly constructed |
| 59 | Quynh Coi | 1 | x | 250 | 250 | Newly constructed |
| 60 | Luc Yen | 1 | x | 125 | 125 | Newly constructed |
| 61 | Song Cong | 1 | x | 250 | 250 | Newly constructed |
| 62 | Bai Bang | 1 | x | 250 | 250 | Newly constructed |
| 63 | Xuan Hoa | 1 | x | 250 | 250 | Newly constructed |
| 64 | Tan Yen | 1 | x | 250 | 250 | Newly constructed |
| 65 | Dong Ky | 2 | x | 250 | 500 | Newly constructed |
| 66 | Bac Ninh | 2 | x | 250 | 500 | In the 500kV subsubstation |
| 67 | Ha Long | 2 | x | 250 | 500 | Newly constructed |
| 68 | Mong Cai | 2 | x | 250 | 500 | Newly constructed |
| 69 | Thuan Chau | 1 | x | 250 | 250 | Newly constructed |
| 70 | Tan Lac | 1 | x | 125 | 125 | Newly constructed |
| 71 | Ngoc Lac | 2 | x | 250 | 500 | Newly constructed |
| 72 | Tinh Gia | 2 | x | 250 | 500 | Newly constructed |
|  | Central area |  |  |  | Central areaCentral areaCentral area5750 |  |
| 1 | Hoa La | 1 | x | 250 | 250 | Install transformer 2 |
| 2 | Ba Don | 1 | x | 250 | 250 | Replace transformer 1 |
| 3 | Le Thuy (Kien Giang) | 1 | x | 250 | 250 | Install transformer 1 |
| 4 | Dong Ha | 1 | x | 250 | 250 | Replace transformer 2 |
| 5 | Lao Bao | 1 | x | 125 | 125 | Install transformer 2 |
| 6 | Phong Dien | 2 | x | 250 | 500 | Replace 2 transformers |
| 7 | Cau Hai | 1 | x | 250 | 250 | Install transformer 2 |
| 8 | Lien Chieu | 1 | x | 125 | 125 | Install transformer 2 |
| 9 | An Don (Tien Sa) | 1 | x | 125 | 125 | Install transformer 2 |
| 10 | Hoi An | 1 | x | 250 | 250 | Install transformer 2 |
| 11 | Tam Ky | 1 | x | 250 | 250 | Replace transformer 2 |
| 12 | Tam Hiep | 1 | x | 250 | 250 | Replace transformer 1 |
| 13 | Bac Chu Lai | 1 | x | 125 | 125 | Install transformer 2 |
| 14 | Dung Quat | 1 | x | 250 | 250 | Replace transformer 2 |
| 15 | Quang Ngai 2 (Nui But) | 1 | x | 125 | 125 | Install transformer 2 |
| 16 | Hoai Nhon | 1 | x | 125 | 125 | Install transformer 2 |
| 17 | Quy Nhon | 1 | x | 250 | 250 | Replace transformer 2 |
| 18 | Nhon Hoi | 1 | x | 250 | 250 | Install transformer 1 |
| 19 | Chi Thanh | 1 | x | 250 | 250 | Install transformer 1 |
| 20 | Cam Ranh | 1 | x | 250 | 250 | Install transformer 2 |
| 21 | Van Phong | 1 | x | 250 | 250 | Install transformer 2 |
| 22 | Bo Y | 1 | x | 125 | 125 | Install transformer 2 |
| 23 | Kon Tum | 1 | x | 125 | 125 | Replace transformer 1 |
| 24 | PleiKu | 1 | x | 250 | 250 | Replace transformer 2 |
| 25 | KrongBuk | 1 | x | 250 | 250 | Replace transformer 2 |
| 26 | Dak Nong | 1 | x | 250 | 250 | Replace transformer 1 |
|  | Southern area |  |  |  | Southern areaSouthern areaSouthern area26250 |  |
| 1 | Bao Loc | 2 | x | 250 | 500 | Replace 02 transformers |
| 2 | Tan Rai | 1 | x | 250 | 250 | Install transformer 2 |
| 3 | Da Lat | 1 | x | 125 | 125 | Install transformer 2 |
| 4 | Ninh Phuoc | 1 | x | 250 | 250 | Install transformer 2 |
| 5 | Du Long | 1 | x | 125 | 125 |  |
| 6 | Phan Thiet | 1 | x | 250 | 250 | Replace transformer |
| 7 | Phan Ri | 1 | x | 250 | 250 | Install transformer 3 |
| 8 | Ham Tan | 1 | x | 250 | 250 | Install transformer 3 |
| 9 | Ham Thuan Nam | 2 | x | 250 | 500 |  |
| 10 | District 8 | 1 | x | 250 | 250 | Install transformer 3 |
| 11 | Tan Tao | 2 | x | 250 | 500 |  |
| 12 | District 2 | 2 | x | 250 | 500 |  |
| 13 | Cu Chi 2 | 2 | x | 250 | 500 |  |
| 14 | District 7 | 2 | x | 250 | 500 |  |
| 15 | Dam Sen | 1 | x | 250 | 250 | Install transformer 3 |
| 16 | Nam Hiep Phuoc | 1 | x | 250 | 250 | Install transformer 3 |
| 17 | Binh Chanh 1 | 1 | x | 250 | 250 | Install transformer 3 |
| 18 | Thu Thiem | 1 | x | 250 | 250 | Install transformer 3 |
| 19 | District 9 | 1 | x | 250 | 250 | Install transformer 3 |
| 20 | Hoc Mon 2 | 2 | x | 250 | 500 |  |
| 21 | Binh Chanh 3 | 2 | x | 250 | 500 |  |
| 22 | Cu Chi 3 | 2 | x | 250 | 500 |  |
| 23 | Binh Long 2 | 2 | x | 250 | 500 |  |
| 24 | Tay Ninh | 1 | x | 250 | 250 | Install transformer 3 |
| 25 | Tay Ninh 2 | 1 | x | 250 | 250 | Install transformer 3 |
| 26 | Tay Ninh 3 | 2 | x | 250 | 500 |  |
| 27 | Lai Uyen | 1 | x | 250 | 250 | Install transformer 3 |
| 28 | Phu Giao | 2 | x | 250 | 500 |  |
| 29 | Nam Dinh 2 | 2 | x | 250 | 500 |  |
| 30 | Go Dau | 2 | x | 250 | 500 |  |
| 31 | An Tay | 1 | x | 250 | 250 | Install transformer 2 |
| 32 | Thu Dau Mot | 2 | x | 250 | 500 |  |
| 33 | My Phuoc 2 | 2 | x | 250 | 500 |  |
| 34 | An Phuoc | 1 | x | 250 | 250 | Install transformer 3 |
| 35 | Tam Phuoc | 1 | x | 250 | 250 | Install transformer 3 |
| 36 | Thong Nhat | 1 | x | 250 | 250 | Install transformer 3 |
| 37 | Long Thanh 2 Industrial Zone | 2 | x | 250 | 500 |  |
| 38 | Dau Giay | 2 | x | 250 | 500 |  |
| 39 | Long Khanh | 2 | x | 250 | 500 |  |
| 40 | Chau Duc | 1 | x | 250 | 250 | Install transformer 3 |
| 41 | Chau Duc 2 | 1 | x | 250 | 250 | Install transformer 2 |
| 42 | Vung Tau 2 | 1 | x | 250 | 250 |  |
| 43 | Phu My City | 1 | x | 250 | 250 | Install transformer 2 |
| 44 | Duc Hoa 3 | 1 | x | 250 | 250 | Install transformer 2 |
| 45 | Duc Hoa 4 | 2 | x | 250 | 500 |  |
| 46 | Can Giuoc | 2 | x | 250 | 500 |  |
| 47 | Thanh Hoa | 1 | x | 250 | 250 |  |
| 48 | Sa Dec Industrial Zone | 1 | x | 250 | 250 | Install transformer 3 |
| 49 | Hong Ngu | 1 | x | 250 | 250 | Install transformer 2 |
| 50 | Thap Muoi | 1 | x | 250 | 250 |  |
| 51 | Cho Moi | 1 | x | 250 | 250 | Install transformer 2 |
| 52 | Phu Tan | 2 | x | 250 | 500 |  |
| 53 | Tri Ton | 2 | x | 250 | 500 |  |
| 54 | Cai Be | 1 | x | 250 | 250 | Install transformer 3 |
| 55 | Cai Be 2 | 1 | x | 250 | 250 | Install transformer 2 |
| 56 | Cho Gao | 2 | x | 250 | 500 |  |
| 57 | Vinh Long 3 | 1 | x | 250 | 250 | Replace transformer 1 |
| 58 | Binh Minh | 1 | x | 250 | 250 |  |
| 59 | Mo Cay | 1 | x | 250 | 250 | Replace transformer 1 |
| 60 | Ba Tri | 1 | x | 250 | 250 | Install transformer 2 |
| 61 | Go Quao | 1 | x | 250 | 250 | Replace transformer 1 |
| 62 | Hon Dat | 1 | x | 250 | 250 | Install transformer 2 |
| 63 | Ha Tien | 2 | x | 250 | 500 |  |
| 64 | O Mon | 1 | x | 250 | 250 | Replace transformer 1 |
| 65 | Ninh Kieu | 1 | x | 250 | 250 | Replace transformer 1 |
| 66 | O Mon 2 | 1 | x | 250 | 250 | Install transformer 2 |
| 67 | Co Do | 2 | x | 250 | 500 |  |
| 68 | Vi Thanh | 1 | x | 250 | 250 | Replace transformer 1 |
| 69 | Phung Hiep | 1 | x | 250 | 250 | Replace transformer 1 |
| 70 | Tra Vinh | 1 | x | 250 | 250 | Replace transformer 1 |
| 71 | Cau Ke | 1 | x | 125 | 125 |  |
| 72 | Soc Trang | 1 | x | 250 | 250 | Replace transformer 1 |
| 73 | My Tu | 1 | x | 250 | 250 | Replace transformer 1 |
| 74 | Long Phu TPP | 2 | x | 250 | 500 | Replace 02 transformers |
| 75 | Bac Lieu 2 | 2 | x | 250 | 500 | Replace 02 transformers |
| 76 | Hong Dan | 1 | x | 125 | 125 | Install transformer 2 |
| 77 | Ngoc Hien | 2 | x | 250 | 500 | Replace 02 transformers |
| 78 | Tran Van Thoi | 1 | x | 250 | 250 | Install transformer 2 |

220kV TRANSMISSION LINES PLANNED TO BE BUILT OVER THE PERIOD FORM 2011 TO 2020

| No | Project name | Number of Circuits x km |  |  | Number of Circuits x kmNumber of Circuits x kmkm | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | Lines built over the period from 2016 to 2020Lines built over the period from 2016 to 2020Lines built over the period from 2016 to 2020Lines built over the period from 2016 to 2020Lines built over the period from 2016 to 2020Lines built over the period from 2016 to 2020 |  |  |  |  |  |
|  | Northern area |  |  |  | 1606 |  |
| 1 | Tay Ha Noi – Hoa Lac | 2 | x | 25 | 50 |  |
| 2 | Soc Son 2 – splitting point Dong Anh – Hiep Hoa | 4 | x | 10 | 40 | 4 circuits in - out |
| 3 | Soc Son 3 – splitting point Hiep Hoa – Yen Phong | 4 | x | 4 | 16 | 4 circuits in - out |
| 4 | Dong Anh 3 – splitting point Dong Anh – Dong Anh 2 | 2 | x | 3 | 6 |  |
| 5 | Long Bien 2 – Long Bien | 2 | x | 7 | 14 |  |
| 6 | Long Bien 2 – Long Bien 500kV substation | 2 | x | 6 | 12 |  |
| 7 | Splitting point – Tay Ha Noi | 2 | x | 8 | 16 |  |
| 8 | Hai Ha Steel – Hai Ha Industrial Zone | 2 | x | 3 | 6 |  |
| 9 | Dai Ban – splitting point Dong Hoa – Pha Lai | 2 | x | 3 | 6 |  |
| 10 | Vinh Bao – splitting point Hai Phong – Thai Binh | 2 | x | 4 | 8 |  |
| 11 | Do Son – splitting point Dong Hoa – Dinh Vu | 2 | x | 6 | 12 |  |
| 12 | Thanh Ha – splitting point Hai Duong TPP – Gia Loc | 2 | x | 7 | 14 |  |
| 13 | Hung Yen HPS – Hung Yen 500kV substation | 2 | x | 16 | 32 |  |
| 14 | Hung Yen city – Ly Nhan | 2 | x | 17 | 34 |  |
| 15 | Hung Yen 500kV substation – Pho Cao | 2 | x | 3 | 6 |  |
| 16 | Thuy Nguyen - Hai Phong 3 TPP | 2 | x | 15 | 30 |  |
| 17 | Hai Phong 3 TPP – splitting point Vat Cach – Trang Bach | 2 | x | 17 | 34 |  |
| 18 | Hai Phong 3 TPP – Dai Ban | 2 | x | 26 | 52 |  |
| 19 | Hai Hau – Truc Ninh | 2 | x | 22 | 44 |  |
| 20 | Hai Hau – Nam Dinh TPP | 2 | x | 8 | 16 |  |
| 21 | Nam Dinh TPP – Ninh Binh 2 | 2 | x | 32 | 64 |  |
| 22 | Nam Dinh TPP - Hau Loc | 2 | x | 37 | 74 |  |
| 23 | An Hoa – splitting point Tien Hai – Truc Ninh | 2 | x | 3 | 6 |  |
| 24 | Tam Diep – splitting point Ninh Binh – Bim Son | 2 | x | 3 | 6 |  |
| 25 | Ninh Binh 2 – splitting point Ninh Binh - Nam Dinh 2 | 2 | x | 7 | 14 |  |
| 26 | Bac Quang – Ha Giang | 2 | x | 58 | 116 |  |
| 27 | Dong Mo – splitting point Lang Son – Bac Giang | 2 | x | 3 | 6 |  |
| 28 | Ham Yen – splitting point Tuyen Quang HPS – Tuyen Quang | 2 | x | 3 | 6 |  |
| 29 | Dai Tu – Thai Nguyen 500kV substation | 2 | x | 26 | 52 |  |
| 30 | Establishing connection to Thai Nguyen 500kV substation | 4 | x | 3 | 12 |  |
| 31 | Pho Vang – splitting point Viet Tri – Son La | 2 | x | 3 | 6 |  |
| 32 | Tam Duong – splitting point Viet Tri – Ba Thien | 2 | x | 3 | 6 |  |
| 33 | Establishing connection to Vinh Yen 500kV substation | 6 | x | 3 | 18 |  |
| 34 | Me Linh - Vinh Yen 500kV substation | 2 | x | 3 | 6 |  |
| 35 | Chu – Bac Gian 500kV substation | 2 | x | 36 | 72 |  |
| 36 | Establishing connection to Bac Giang 500kV substation | 4 | x | 3 | 12 |  |
| 37 | Vo Cuong - Bac Ninh 500kV substation | 2 | x | 16 | 32 |  |
| 38 | Thuan Thanh - Bac Ninh 500kV substation | 2 | x | 12 | 24 |  |
| 39 | Nong Cong – splitting point Thanh Hoa – Nghi Son | 2 | x | 3 | 6 |  |
| 40 | Establish connection to Thanh Hoa 500kV substation | 6 | x | 3 | 18 |  |
| 41 | Tuong Duong – splitting point Ban Ve – Do Luong | 2 | x | 3 | 6 |  |
| 42 | Nam Thean 1 – Ha Tinh | 2 | x | 280 | 560 |  |
| 43 | Hoang Mai – Nghi Son TPP | 2 | x | 15 | 30 |  |
| 44 | Can Loc - splitting point Vinh - Ha Tinh | 2 | x | 3 | 6 |  |
|  | Central area |  |  |  | 1362 |  |
| 1 | Splitting point Cua Tung – Cua Tung | 2 | x | 4 | 8 |  |
| 2 | Splitting point Hon La – Hon La | 2 | x | 5 | 10 |  |
| 3 | Hoa Khanh – Da Nang 2 | 2 | x | 8 | 16 |  |
| 4 | Establishing connection to Central CPPs | 2 | x | 300 | 600 |  |
| 5 | Establishing connection to Central Gas turbine power plant | 2 | x | 300 | 600 |  |
| 6 | An Don – Da Nang 2 | 2 | x | 8 | 16 |  |
| 7 | Splitting point Bac Chu Lai – Bac Chu Lai | 4 | x | 5 | 20 |  |
| 8 | Splitting point Quang Ngai 2 – Quang Ngai 2 | 4 | x | 3 | 12 |  |
| 9 | Establishing connection to Binh Dinh TPP | 4 | x | 20 | 80 |  |
|  | Southern area |  |  |  | 2584 |  |
| 1 | Establishing connection to Southern Gas turbine power plant | 2 | x | 200 | 400 |  |
| 2 | Establishing connection to Southern CPPs | 2 | x | 600 | 1200 |  |
| 3 | Da Nhim – Da Lat | 2 | x | 33 | 66 |  |
| 4 | Du Long - splitting point Cam Ranh – Thap Cham | 4 | x | 2 | 8 |  |
| 5 | Dinh Quan – splitting point Song May - Bao Loc | 4 | x | 2 | 8 |  |
| 6 | Tan Duc - Ham Tan | 2 | x | 24 | 48 |  |
| 7 | Dat Do – splitting point Ham Tan – Chau Duc | 4 | x | 1 | 4 |  |
| 8 | Dat Do – Vung Tau 2 | 2 | x | 18 | 36 |  |
| 9 | Vung Tau 2 – splitting point Ba Ria – Vung Tau | 4 | x | 1 | 4 |  |
| 10 | Dong Nai 500kV substation – splitting point Phu My 3 industrial zone | 2 | x | 23 | 46 |  |
| 11 | Phu My city - Splitting point Phu My - Phu My 2 industrial zone | 4 | x | 2 | 8 |  |
| 12 | Thong Nhat – splitting point Song May - Bao Loc | 4 | x | 2 | 8 |  |
| 13 | Nhon Trach city - Nhon Trach Industrial Zone | 2 | x | 4 | 8 |  |
| 14 | Nhon Trach Industrial Zone – Long Thanh 500kV substation | 2 | x | 12 | 24 |  |
| 15 | Phuoc An CCPP - Nhon Trach city | 2 | x | 10 | 20 |  |
| 16 | Phuoc An CCPP - Fomosa | 2 | x | 12 | 24 |  |
| 17 | Eastern District 9 – splitting point Long Thanh 500kv substation –Thu Duc | 4 | x | 1 | 4 |  |
| 18 | Tam Binh – splitting point Tan Uyen 500kV substation – Hoc Mon | 4 | x | 1 | 4 |  |
| 19 | Tan Uyen 500kV substation – splitting point Hoc Mon | 2 | x | 14 | 28 | Using same 4-circuit towers of Tan Uyen - Hiep Binh Phuoc line |
| 20 | Tan Thoi Hiep - splitting point Thuan An - Cau Bong | 4 | x | 2 | 8 |  |
| 21 | Tay Bac Cu Chi – splitting point Duc Hoa 500kV substation - Cu Chi | 4 | x | 4 | 16 |  |
| 22 | Duc Hoa 2 – splitting point Duc Hoa - Cu Chi | 2 | x | 3 | 6 |  |
| 23 | Duc Hoa 3 – splitting point Duc Hoa - Cu Chi | 2 | x | 3 | 6 |  |
| 24 | Cu Chi 500kV substation – connect to existing line | 10 | x | 1 | 10 |  |
| 25 | Phu Hoa Dong – splitting point Cau Bong 500kV substation – Cu Chi | 4 | x | 2 | 8 |  |
| 26 | My Phuoc 500kV substation – splitting point Tan Dinh 2 – Cu Chi | 4 | x | 11 | 44 | Using 4-circuit towers |
| 27 | An Tay – splitting point My Phuoc 500kV substation – Cu Chi | 4 | x | 2 | 8 |  |
| 28 | Long An TPP – Can Duoc | 2 | x | 11 | 22 |  |
| 29 | Long An TPP – splitting point Go Cong | 2 | x | 2 | 4 |  |
| 30 | My Tho 500kV substation - My Tho | 2 | x | 12 | 24 | Circuits 3,4 |
| 31 | Ben Tre – Ba Tri | 2 | x | 31 | 62 |  |
| 32 | Cai Be – splitting point Cai Lay – Cao Lanh | 2 | x | 2 | 4 |  |
| 33 | Chau Doc – Hong Ngu | 2 | x | 32 | 64 |  |
| 34 | Hong Ngu – Thanh Binh | 2 | x | 30 | 60 |  |
| 35 | Thai Binh – Cai Be | 2 | x | 62 | 124 |  |
| 36 | Cho Moi – splitting point Thanh Binh – Hong Ngu | 2 | x | 10 | 20 |  |
| 37 | O Mon 2 – splitting point O Mon - Thot Not | 4 | x | 2 | 8 |  |
| 38 | Hon Dat – splitting point Kien Luong – Rach Gia | 2 | x | 2 | 4 |  |
| 39 | Kien Luong TPP – Ha Tien | 2 | x | 23 | 46 |  |
| 40 | Hong Dan – splitting point Gia Rai – Bac Lieu 2 | 2 | x | 16 | 32 |  |
| 41 | Ca Mau TPP – Tran Van Thoi | 2 | x | 28 | 56 |  |
|  | Lines built over the period from 2026 to 2030Lines built over the period from 2026 to 2030Lines built over the period from 2026 to 2030Lines built over the period from 2026 to 2030Lines built over the period from 2026 to 2030Lines built over the period from 2026 to 2030 |  |  |  |  |  |
|  | Northern area |  |  |  | 2168 |  |
| 1 | Phu Xuyen – Southern Ha Noi 500kV substation | 2 | x | 19 | 38 |  |
| 2 | Establishing connection to Northern CPP | 2 | x | 600 | 1200 |  |
| 3 | Xuan Mai urban center – Southern Ha Noi 500kV substation | 2 | x | 22 | 44 |  |
| 4 | Phu Xuyen – Hung Yen city | 2 | x | 15 | 30 |  |
| 5 | Hoa Lac 2 – Hoa Lac | 2 | x | 12 | 24 |  |
| 6 | Hoa Lac 2 – Son Tay 500kV substation | 2 | x | 22 | 44 |  |
| 7 | Connection – Son Tay 500kv substation | 4 | x | 3 | 12 |  |
| 8 | Long Bien 3 – Long Bien 500kV substation | 2 | x | 4 | 8 |  |
| 9 | Tien Lang - Hai Phong 500kV line | 2 | x | 14 | 28 |  |
| 10 | Thuy Nguyen – splitting point Vat Cach – Trang Bach | 2 | x | 2 | 4 |  |
| 11 | Establishing connection to Hai Ha TPP | 4 | x | 80 | 320 |  |
| 12 | Tu Ky - Hai Phong 500kV substation | 2 | x | 14 | 28 |  |
| 13 | Tu Ky – Gia Loc | 2 | x | 20 | 40 |  |
| 14 | Thanh Mien – Hung Yen 500kV substation | 2 | x | 12 | 24 |  |
| 15 | Tien Hai – splitting point Vu Thu – Truc Ninh | 2 | x | 5 | 10 |  |
| 16 | Khoai Chau - splitting point Thuong Tin – Kim Dong | 2 | x | 3 | 6 |  |
| 17 | Van Giang - Long Bien 500kV substation | 2 | x | 4 | 8 |  |
| 18 | Dong Van – Southern Ha Noi 500kV substation | 2 | x | 15 | 30 |  |
| 19 | Quynh Coi – splitting point Thai Binh – Pho Cao | 2 | x | 3 | 6 |  |
| 20 | Luc Yen – splitting point Yen Bai – Bao Thang | 2 | x | 3 | 6 |  |
| 21 | Song Cong – splitting point Hiep Hoa - Thai Nguyen | 2 | x | 3 | 6 |  |
| 22 | Bai Bang – Viet Tri 500kV substation | 2 | x | 12 | 24 |  |
| 23 | Xuan Hoa – Vinh Yen 500kV substation | 2 | x | 2 | 4 |  |
| 24 | Me Linh – Vinh Yen 500kV substation | 2 | x | 3 | 6 |  |
| 25 | Tan Yen – Bac Giang 500kV substation | 2 | x | 22 | 44 |  |
| 26 | Dong Ky - Dong Anh 500kV substation | 2 | x | 7 | 14 |  |
| 27 | Ha Long – Quang Ninh | 2 | x | 14 | 28 |  |
| 28 | Mong Cai –Hai Ha TPP | 2 | x | 40 | 80 |  |
| 29 | Thuan Chau – splitting point Son La – Dien Bien | 2 | x | 3 | 6 |  |
| 30 | Ngoc Lac – splitting point Thanh Hoa – Cua Dat | 2 | x | 20 | 40 |  |
| 31 | Tinh Gia - splitting point Thanh Hoa – Nghi Son | 2 | x | 3 | 6 |  |
|  | Central area |  |  |  | 916 |  |
| 1 | Establishing connection to Central CPPs | 2 | x | 200 | 400 |  |
| 2 | Establishing connection to Central NPPs | 2 | x | 200 | 400 |  |
| 3 | Splitting point Kien Giang – Kien Giang | 4 | x | 5 | 20 |  |
| 4 | Splitting point Chi Thanh – Chi Thanh | 4 | x | 4 | 16 |  |
| 5 | Binh Dinh TPP – Nhon Hoi | 2 | x | 40 | 80 |  |
|  | Southern area |  |  |  | 1936 |  |
| 1 | Establishing connection to Southern Gas turbine power plant | 2 | x | 200 | 400 |  |
| 2 | Establishing connection to Southern CPPs | 2 | x | 400 | 800 |  |
| 3 | Ham Thuan Nam – splitting point Phan Thiet - Ham Tan | 4 | x | 2 | 8 |  |
| 4 | Dong Nai 500kV substation – splitting point Xuan Loc – Long Thanh | 4 | x | 19 | 76 | Using 4-circuit towers |
| 5 | Long Thanh Industrial Zone - Dong Nai 500kV substation – Long Thanh | 4 | x | 1 | 4 | Using 4-circuit towers |
| 6 | Long Khanh – splitting point Dong Nai 500kV substation – Xuan Loc | 4 | x | 1 | 4 |  |
| 7 | District 7 – Nha Be | 2 | x | 5 | 10 | Install underground 220kV line |
| 8 | Long Thanh 500kV substation – Dau Giay | 2 | x | 20 | 40 |  |
| 9 | Go Dau – splitting point Tan Dinh – Binh Hoa | 4 | x | 2 | 8 |  |
| 10 | Thu Dau 1 – Tan Dinh 500kV substation | 2 | x | 5 | 10 |  |
| 11 | My Phuoc 2 – Binh Duong 1 500kV substation | 2 | x | 12 | 24 |  |
| 12 | Phu Giao – Binh Duong 1 500kV substation | 2 | x | 18 | 36 |  |
| 13 | Dinh An - My Phuoc 500kV substation – Tan Dinh 2 | 4 | x | 2 | 8 |  |
| 14 | Cu Chi 3 – splitting point Phu Hoa Dong –Cau Bong | 4 | x | 2 | 8 |  |
| 15 | Hoc Mon 2 – splitting point Binh Chanh 1 - Cau Bong | 4 | x | 1 | 4 |  |
| 16 | Duc Hoa 500kV substation – Duc Hoa 500kV substation | 2 | x | 30 | 60 | For circuits 3,4 |
| 17 | Cu Chi 2 – splitting point Duc Hoa - Cu Chi | 4 | x | 1 | 4 |  |
| 18 | Duc Hoa 4 – splitting point Duc Hoa - Cu Chi | 4 | x | 4 | 16 |  |
| 19 | Duc Hoa 500kV line – Binh Chanh 3 | 2 | x | 9 | 18 |  |
| 20 | Binh Chanh 3 – Tan Tao | 2 | x | 7 | 14 |  |
| 21 | Tan Tao – splitting point Binh Chanh 2 | 2 | x | 6 | 12 |  |
| 22 | Duc Hoa CCPP – splitting point Binh Chanh 2 – Long An | 4 | x | 13 | 52 |  |
| 23 | Tay Ninh 500kV substation – splitting point Tay Ninh 2 – Tay Ninh | 4 | x | 2 | 8 |  |
| 24 | Tay Ninh 500kV substation – splitting point Binh Long | 2 | x | 1 | 2 |  |
| 25 | Tay Ninh 500kV substation – Tay Ninh 3 | 2 | x | 17 | 34 |  |
| 26 | Binh Long 2 – splitting point Binh Long - Lai Uyen | 4 | x | 2 | 8 |  |
| 27 | Long An TPP – Can Giuoc | 2 | x | 11 | 22 |  |
| 28 | Cho Gao – splitting point Long An – My Tho 500kV substation | 4 | x | 2 | 8 |  |
| 29 | Thanh Hoa - My Tho 500kV substation | 2 | x | 26 | 52 |  |
| 30 | Cai Be 2 – splitting point Cai Be – Cai Lay | 2 | x | 2 | 4 |  |
| 31 | Tien Giang 500kV substation – splitting point Thanh Binh – Cai Be | 2 | x | 2 | 4 |  |
| 32 | Tien Giang 500kV substation – splitting point Cao Lanh – Cai Be 2 | 2 | x | 2 | 4 |  |
| 33 | Tien Giang 500kV substation – Binh Minh | 2 | x | 25 | 50 |  |
| 34 | Thap Muoi - splitting point Thanh Binh – Cai Be | 4 | x | 2 | 8 |  |
| 35 | Phu Tan – splitting point Hong Ngu – Chau Doc | 4 | x | 1 | 4 |  |
| 36 | Tri Ton – splitting point Kien Luong – Chau Doc | 4 | x | 10 | 40 |  |
| 37 | Co Do – splitting point Ca Mau – O Mon | 4 | x | 2 | 8 |  |
| 38 | Cau Ke – splitting point Tra Vinh – Vinh Long 3 | 2 | x | 12 | 24 |  |
| 39 | Cho Moi – Chau Thanh | 2 | x | 14 | 28 |  |
| 40 | Dong Thap 500kV substation – splitting point Thanh Binh – Hong Ngu | 4 | x | 2 | 8 |  |
| 41 | Dong Thap 500kV substation – splitting point Cho Moi | 2 | x | 2 | 4 |  |